|  | INTERNATIONAL TELECOMMUNICATION UNION |
| --- | --- |

ITU-T	Q.763

TELECOMMUNICATION	(03/93)
STANDARDIZATION  SECTOR
OF  ITU

SPECIFICATIONS  OF  SIGNALLING  SYSTEM  No.  7

FORMATS  AND  CODES  OF  THE  ISDN  USER  PART  OF  SIGNALLING  SYSTEM  No.  7

ITU-T  Recommendation  Q.763

(Previously “CCITT  Recommendation”)

CONTENTS

Recommendation Q.763     (03/93)Page

1	General		1

2	Parameter formats and codes		6

3	ISDN User Part parameters		8

4	ISDN user part messages and codes		60

Annex A  –  Interpretation of spare codes		71

Annex B  –  General description of component encoding rules		81

Recommendation Q.763

Recommendation Q.763     (03/93)

FORMATS  AND  CODES1)  OF  THE  ISDN  USER  PART
OF  SIGNALLING  SYSTEM  No.  7

(Malaga-Torremolinos, 1984; modified at Helsinki, 1993)

# 1	General

ISDN User Part messages are carried on the signalling link by means of signal units, the format of which is described in 2.2/Q.703.

The format of and the codes used in the service information octet are described in 14.2/Q.704. The service indicator for the ISDN User Part is coded 0101.

The signalling information field of each message signal unit containing an ISDN User Part message consists of an integral number of octets and encompasses the following parts (see Figure 1):

a)	routing label;

b)	circuit identification code;

c)	message type code;

d)	the mandatory fixed part;

e)	the mandatory variable part;

f)	the optional part, which may contain fixed length and variable length parameter fields.

NOTE – The service information octet, the routing label and circuit identification code are not included in the SCCP user data parameter transferred between the ISDN User Part and signalling connection control part.

A description of the various message parts is given in the following subclauses.

FIGURE  1/Q.763

Routing label

Circuit identification code

Message type code

Mandatory fixed part

Mandatory variable part

Optional part

ISDN User Part message parts

## 1.1	Routing label

The format and codes used for the routing label are described in 2.2/Q.704. For each individual circuit connection, the same routing label must be used for each message that is transmitted for that connection.

NOTE – The SLS bits are set to the four least significant bits of the CIC.

## 1.2	Circuit identification code

The format of the circuit identification code (CIC) is shown in Figure 2.

FIGURE  2/Q.763

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Circuit identification code (least significant bits) |  |  |  |  |  |  |  |
| Spare |  |  |  | CIC most significant bits |  |  |  |

Circuit identification field

## 1.4	Formatting principles

Each message consists of a number of PARAMETERS listed and described in 2. Each parameter has a NAME which is coded as a single octet (see Table 5). The length of a parameter may be fixed or variable, and a LENGTH INDICATOR of one octet for each parameter may be included as described below.

The detailed format is uniquely defined for each message type as described in 3.

Between parameters there should be no unused (i.e. dummy) octets.

A general format diagram is shown in Figure 3.

## 1.5	Mandatory fixed part

Those parameters that are mandatory and of fixed length for a particular message type will be contained in the mandatory fixed part. The position, length and order of the parameters is uniquely defined by the message type, thus the names of the parameters and the length indicators are not included in the message.

## 1.6	Mandatory variable part

Mandatory parameters of variable length will be included in the mandatory variable part. Pointers are used to indicate the beginning of each parameter. Each pointer is encoded as a single octet. The name of each parameter and the order in which the pointers are sent is implicit in the message type. Parameter names are, therefore, not included in the message. The details of how pointers are encoded is found in 2.3. The number of parameters, and thus the number of pointers is uniquely defined by the message type.

A pointer is also included to indicate the beginning of the optional part. If the message type indicates that no optional part is allowed, then this pointer will not be present. If the message type indicates that an optional part is possible (reflected by the presence of an “end of optional parameter” octet in Tables 4 through 42), but there is no optional part included in this particular message, then a pointer field containing all zeros will be used. It is recommended that all future message types with a mandatory variable part indicate that an optional part is allowed.

All the pointers are sent consecutively at the beginning of the mandatory variable part. Each parameter contains the parameter length indicator followed by the contents of the parameters. If there are no mandatory variable parameters, but optional parameters are possible, the start of optional parameters pointer (coded all “0”s if no optional parameter is present and coded “00000001” if any optional parameter is present) will be included.

## 1.7	Optional part

The optional part consists of parameters that may or may not occur in any particular message type. Both fixed length and variable length parameters may be included. Unless it is explicitly stated to the contrary within the ISUP 1992 Recommendations, an optional parameter cannot occur multiple times within one message. Optional parameters may be transmitted in any order. Each optional parameter will include the parameter name (one octet) and the length indicator (one octet) followed by the parameter contents.

## 1.8	End of optional parameters octet

If optional parameters are present and after all optional parameters have been sent, an “end of optional parameters” octet containing all zeros will be transmitted. If no optional parameter is present an “end of optional parameter” octet is not transmitted.

## 1.9	Order of transmission

Since all the fields consist of an integral number of octets, the formats are presented as a stack of octets. The first octet transmitted is the one shown at the top of the stack and the last is the one at the bottom (see Figure 3).

Unless otherwise indicated, within each octet and subfield the bits are transmitted with the least significant bit first.

## 1.10	Coding of spare bits

Spare bits are coded 0 unless indicated otherwise.

## 1.11	National message types and parameters

If message type codes and parameter name codes are required for national uses not included in this Recommendation, the codes chosen should be from the highest code downwards, that is, starting at code 11111111. Codes in the range 11111111 to 11100000 are reserved exclusively for this purpose.

# 2	Parameter formats and codes

## 2.1	Message type codes

The encoding of the message type is shown in Table 4.

## 2.2	Coding of the length indicator

The length indicator field is binary coded to indicate the number of octets in the parameter content field. The length indicated does not include the parameter name octet or the length indicator octet.

TABLE  4/Q.763

| Тип сообщения Message type (МТ) | Ссылка на табл. N | Мнемоника | Код МТ (НЕХ) | Код МТ (BIN) |
| --- | --- | --- | --- | --- |
| Address complete (Сообщение полного адреса) | 21 | ACM | 06 | 0000 0110 |
| Answer (Сообщение “Ответ абонента”) | 22 | ANM | 09 | 0000 1001 |
| Blocking (Сообщение о блокировке канала) | 39 | BLO | 13 | 0001 0011 |
| Blocking acknowledgement (Подтверждение блокировки канала) | 39 | BLA | 15 | 0001 0101 |
| Call progress (Сообщение “Вызов продолжается”) | 23 | CPG | 2C | 0010 1100 |
| Circuit group blocking (Блокировка группы каналов) | 40 | CGB | 18 | 0001 1000 |
| Circuit group blocking acknowledgement | 40 | CGBA | 1A | 0001 1010 |
| Circuit group query @ | 41 | CQM | 2A | 0010 1010 |
| Circuit group query response @ | 24 | CQR | 2B | 0010 1011 |
| Circuit group reset (Сброс группы каналов) | 41 | GRS | 17 | 0001 0111 |
| Circuit group reset acknowledgement | 25 | GRA | 29 | 0010 1001 |
| Circuit group unblocking (Разблокировка группы каналов) | 40 | CGU | 19 | 0001 1001 |
| Circuit group unblocking acknowledgement | 40 | CGUA | 1B | 0001 1011 |
| Charge information @ (Информация о тарификации - таксации) | (Note) | CRG | 31 | 0011 0001 |
| Confusion | 26 | CFN | 2F | 0010 1111 |
| Connect (Соединение) | 27 | CON | 0F | 0000 0111 |
| Continuity (Целостность) | 28 | COT | 05 | 0000 0101 |
| Continuity check request (Запрос проверки целостности) | 39 | CCR | 11 | 0001 0001 |
| Facility @ (Предоставление услуги) | 45 | FAC | 33 | 0011 0011 |
| Facility accepted (Услуга принята) | 42 | FAA | 20 | 0010 0000 |
| Facility reject (Услуга отклонена) | 29 | FRJ | 21 | 0010 0001 |
| Facility request (Запрос услуги) | 42 | FAR | 1F | 0001 1111 |
| Forward transfer (Прямая передача) | 37 | FOT | 08 | 0000 1000 |
| Identification request (Запрос идентификации) | 47 | IDR | 36 | 0011 0110 |
| Identification response (ответ на запрос идентификации) | 48 | IRS | 37 | 0011 0111 |
| Information @ (Информационное сообщение) | 30 | INF | 04 | 0000 0100 |
| Information request @ (Запрос информации) | 31 | INR | 03 | 0000 0011 |
| Initial address (Сообщение начального адреса) | 32 | IAM | 01 | 0000 0001 |
| Loop back acknowledgement @ | 39 | LPA | 24 | 0010 0100 |
| Network resource management | 46 | NRM | 32 | 0011 0010 |
| Overload @ (Сообщение о перегрузке) | 39 | OLM | 30 | 0011 0000 |
| Pass-along @ (Сквозная передача) | 43 | PAM | 28 | 0010 1000 |
| Release (Сообщение о разъединении) | 33 | REL | 0C | 0000 1100 |
| Release complete (Подтверждение разъединения) | 34 | RLC | 10 | 0001 0000 |
| Reset circuit (Сброс канала) | 39 | RSC | 12 | 0001 0010 |
| Resume (Возобновление) | 38 | RES | 0E | 0000 1110 |
| Segmentation | 49 | SGM | 38 | 0011 1000 |
| Subsequent address (Последующий адрес) | 35 | SAM | 02 | 0000 0010 |
| Suspend (Пауза) | 38 | SUS | 0D | 0000 1101 |
| Unblocking (Разблокировка каналов) | 39 | UBL | 14 | 0001 0100 |
| Unblocking acknowledgement (Подтверждение разблокировки) | 39 | UBA | 16 | 0001 0110 |
| Unequipped CIC @ (Незадействованный канал - CIC) | 39 | UCIC | 2E | 0010 1110 |
| User Part available (Подсистема пользователя недоступна) | 44 | UPA | 35 | 0011 0101 |
| User Part test (Тестирование подсистемы пользователей) | 44 | UPT | 34 | 0011 0100 |
| User-to-user information (Информация от пользователя к пользователю) | 36 | USR | 2D | 0010 1101 |
| Reserved (used in 1984 version) |  |  | 0A 0B 0F 23 25 26 | 0000 1010 0000 1011 0000 1111 0010 0010 0010 0011 0010 0101 0010 0110 |
| Reserved (used in 1988 version) |  |  | 1D 1C 1E 27 | 0001 1101 0001 1100 0001 1110 0010 0111 |

## 2.3	Coding of the pointers

The pointer value (in binary) gives the number of octets between the pointer itself (included) and the first octet (not included) of the parameter associated with that pointer.

The pointer value all zeros is used to indicate that, in the case of optional parameters, no optional parameter is present.

# 3	ISDN User Part parameters

## 3.1	Parameter names

The parameter name codes are given in Table 5 together with references to the subclauses in which they are described.

TABLE  5/Q.763

| Parameter name (Имя параметра) |  | Ссылка на подраздел |  | Code (HEX) |  | Code (DEC) |
| --- | --- | --- | --- | --- | --- | --- |
| Access delivery information |  | 3.2 |  | 2 E |  | 0010 1110 |
| Access transport |  | 3.3 |  | 0 3 |  | 0000 0011 |
| Automatic congestion level |  | 3.4 |  | 2 7 |  | 0010 0111 |
| Backward call indicators |  | 3.5 |  | 1 1 |  | 0001 0001 |
| Call diversion information |  | 3.6 |  | 3 6 |  | 0011 0110 |
| Call history information |  | 3.7 |  | 2 D |  | 0010 1101 |
| Call reference @ |  | 3.8 |  | 0 1 |  | 0000 0001 |
| Called party number (Номер вызываемого абонента) |  | 3.9 |  | 0 4 |  | 0000 0100 |
| Calling party number (Номер вызывающего абонента) |  | 3.10 |  | 0 A |  | 0000 1010 |
| Calling party’s category (Категория вызывающего абонента) |  | 3.11 |  | 0 9 |  | 0000 1001 |
| Circuit state indicator @ |  | 3.14 |  | 2 6 |  | 0010 0110 |
| Cause indicators (Индикаторы причины – см. Рек. Q.850) |  | 3.12 |  | 1 2 |  | 0001 0010 |
| Circuit group supervision message type indicator |  | 3.13 |  | 1 5 |  | 0001 0101 |
| Closed user group interlock code |  | 3.15 |  | 1 A |  | 0001 1010 |
| Connected number |  | 3.16 |  | 2 1 |  | 0010 0001 |
| Connection request |  | 3.17 |  | 0 D |  | 0000 1101 |
| Continuity indicators |  | 3.18 |  | 1 0 |  | 0001 0000 |
| Echo control information |  | 3.19 |  | 3 7 |  | 0011 0111 |
| End of optional parameters |  | 3.20 |  | 0 0 |  | 0000 0000 |
| Event information |  | 3.21 |  | 2 4 |  | 0010 0100 |
| Facility indicator |  | 3.22 |  | 1 8 |  | 0001 1000 |
| Forward call indicators |  | 3.23 |  | 0 7 |  | 0000 0111 |
| Freephone indicators (reserved) |  | (Note) |  | 4 1 |  | 0100 0001 |
| Generic digits @ |  | 3.24 |  | C 1 |  | 1100 0001 |
| Generic notification |  | 3.25 |  | 2 C |  | 0010 1100 |
| Generic number |  | 3.26 |  | C 0 |  | 1100 0000 |
| Generic reference (reserved) |  | 3.27 |  | 4 2 |  | 0100 0010 |
| Hop counter (reserved) |  | (Note) |  | 3 D |  | 0011 1101 |
| Information indicators @ |  | 3.28 |  | 0 F |  | 0000 1111 |
| Information request indicators |  | 3.29 |  | 0 E |  | 0000 1110 |
| Location number |  | 3.30 |  | 3 F |  | 0011 1111 |
| MCID request indicator |  | 3.31 |  | 3 B |  | 0011 1011 |
| MCID response indicator |  | 3.32 |  | 3 C |  | 0011 1100 |
| Message compatibility information |  | 3.33 |  | 3 8 |  | 0011 1000 |
| MLPP precedence |  | 3.34 |  | 3 A |  | 0011 1010 |
| Nature of connection indicators |  | 3.35 |  | 0 6 |  | 0000 0110 |
| Network specific facilities @ |  | 3.36 |  | 2 F |  | 0010 1111 |
| Optional backward call indicators |  | 3.37 |  | 2 9 |  | 0010 1001 |
| Optional forward call indicators |  | 3.38 |  | 0 8 |  | 0000 1000 |
| Original called number |  | 3.39 |  | 2 8 |  | 0010 1000 |
| Origination ISC point code |  | 3.40 |  | 2 B |  | 0010 1011 |
| Parameter compatibility information |  | 3.41 |  | 3 9 |  | 0011 1001 |
| Propagation delay counter |  | 3.42 |  | 3 1 |  | 0011 0001 |
| Range and status |  | 3.43 |  | 1 6 |  | 0001 0110 |
| Redirecting number |  | 3.44 |  | 0 B |  | 0000 1011 |
| Redirection information |  | 3.45 |  | 1 3 |  | 0001 0011 |
| Redirection number |  | 3.46 |  | 0 C |  | 0000 1100 |
| Redirection number restriction |  | 3.47 |  | 4 0 |  | 0100 0000 |
| Remote operations @ |  | 3.48 |  | 3 2 |  | 0011 0010 |
| Service activation @ |  | 3.49 |  | 3 3 |  | 0011 0011 |
| Signalling point code @ |  | 3.50 |  | 1 E |  | 0001 1110 |
| Subsequent number |  | 3.51 |  | 0 5 |  | 0000 0101 |
| Suspend/Resume indicators |  | 3.52 |  | 2 2 |  | 0010 0010 |
| Transit network selection @ |  | 3.53 |  | 2 3 |  | 0010 0011 |
| Transmission medium requirement |  | 3.54 |  | 0 2 |  | 0000 0010 |
| Transmission medium requirement prime |  | 3.55 |  | 3 E |  | 0011 1110 |
| Transmission medium used | 3.56 |  | 3 5 |  | 0011 0101 |  |
| User service information | 3.57 |  | 1 D |  | 0001 1101 |  |
| User service information prime | 3.58 |  | 3 0 |  | 0011 0000 |  |
| User teleservice information | 3.59 |  | 3 4 |  | 0011 0100 |  |
| User-to-user indicators | 3.60 |  | 2 A |  | 0010 1010 |  |
| User-to-user information | 3.61 |  | 2 0 |  | 0010 0000 |  |
| Reserved (used in 1984 version, Red Book) Reserved for multi-slot identifier |  |  | 1 4 1 9 1 B 1 C 1 F 2 5 |  | 0001 0100 0001 1001 0001 1011 0001 1100 0001 1111 0010 0101 |  |
| Reserved (used in 1988 version, Blue Book) |  |  | 1 7 |  | 0001 0111 |  |
| NOTE – The format is not provided for the time being. |  |  |  |  |  |  |

## 3.2	Access delivery information (open)

The format of the access delivery information parameter field is shown in Figure 4.

FIGURE  4/Q.763

|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | H | G | F | E | D | C | B | A |

Acces delivery information parameter field

bit	A	Access delivery indicator
1	No set-up message generated
0	Set-up message generated

bits H-B	not used

NOTE – Not used bits in the access delivery information parameter field can be used in the future for additional information concerning access delivery aspects.

## 3.3	Access transport

The format of the access transport parameter field is shown in Figure 5.

FIGURE  5/Q.763

|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 |  |  |  |  |  |  |  |  |
| · · · | Information element(s) |  |  |  |  |  |  |  |
| n |  |  |  |  |  |  |  |  |

Access transport parameter field

The information element is coded as described in 4.5/Q.931. Multiple Q.931 information elements can be included within the access transport parameter. The information elements applicable to a particular usage of the access transport parameter are dependent on, and will be determined by, the relevant procedures. The maximum length of the access transport parameter should only be limited by the message length as the content of the ATP will probably evolve in the future.

## 3.4	Automatic congestion level

The format of the automatic congestion level parameter field is shown in Figure 6.

FIGURE  6/Q.763

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Automatic congestion level |  |  |  |  |  |  |  |

Automatic congestion level parameter field

The following codes are used in the automatic congestion level parameter field:

00000000	Spare
00000001	Congestion level 1 exceeded
00000010	Congestion level 2 exceeded

Spare

## 3.5	Backward call indicators

The format of the backward call indicators parameter field is shown in Figure 7.

FIGURE  7/Q.763

|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | H | G | F | E | D | C | B | A |
| 2 | P | O | N | M | L | K | J | I |

Backward call indicators parameter field

The following codes are used in the backward call indicators parameter field:

bits	B	A:	Charge indicator
0	0	no indication
0	1	no charge
1	0	charge
1	1	spare

NOTE 1 – The interpretation of these bits depends only on the charging exchange.

bits	D	C:	Called party’s status indicator
0	0	no indication
0	1	subscriber free
1	0	connect when free @
1	1	spare

bits	F	E:	Called party’s category indicator
0	0	no indication
0	1	ordinary subscriber
1	0	payphone
1	1	spare

bits	H	G:	End-to-end method indicator (Note 2)
0	0	no end-to-end method available (only link-by-link method available)
0	1	pass along method available @
1	0	SCCP method available
1	1	pass along and SCCP methods available @

bit	I:		Interworking indicator (Note 2)
0		no interworking encountered
1		interworking encountered

bit	J:		End-to-end information indicator (Note 2) @
0		no end-to-end information available
1		end-to-end information available

bit	K:		ISDN User Part indicator (Note 2)
0		ISDN User Part not used all the way
1		ISDN User Part used all the way

bit	L:		Holding indicator (national use) @
0		holding not requested
1		holding requested

bit	M:		ISDN access indicator
0		terminating access non-ISDN
1		terminating access ISDN

bit	N:		Echo control device indicator
0		incoming half echo control device not included
1		incoming half echo control device included

bits	P	O:	SCCP method indicator (Note 2)
0	0	no indication
0	1	connectionless method available @
1	0	connection oriented method available
1	1	connectionless and connection oriented methods available @

NOTE 2 – Bits G-K and O-P constitute the protocol control indicator.

## 3.6	Call diversion information

The format of the call diversion information parameter field is shown in Figure 8.

FIGURE  8/Q.763

|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | H | G | F | E | D | C | B | A |

Call diversion information parameter field

The following codes are used in the call diversion information parameter field:

a)	Notification subscription options

bits	CBA:	000	Unknown
	001	presentation not allowed
	010	presentation allowed with redirection number
	011	presentation allowed without redirection number

spare

b)	Redirection reason

bits GFED:	0000	Unknown
	0001	User busy
	0010	no reply
	0011	unconditional
	0100	deflection during alerting
	0101	deflection immediate response
	0110	mobile subscriber not reachable

spare

bit H:	Spare

## 3.7	Call history information

The format of the call history information parameter field is shown in Figure 42.

The call history information parameter expresses in pure binary representation the propagation delay value of a call in ms.

## 3.8	Call reference @

The format of the call reference parameter is shown in Figure 9.

FIGURE  9/Q.763

|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 |  |  |  |  |  |  |  |  |
| 2 | Call identity |  |  |  |  |  |  |  |
| 3 |  |  |  |  |  |  |  |  |
| 4 |  |  |  |  |  |  |  |  |
| 5 | Spare |  | Point code |  |  |  |  |  |

Call reference parameter field

The following codes are used in the subfields of the call reference parameter field:

a)	Call identity

A code expressing in pure binary representation the identification number allocated to the call.

b)	Point code

The code of the signalling point in which the call identity is relevant.

## 3.9	Called party number

The format of the called party number parameter field is shown in Figure 10.

FIGURE  10/Q.763

|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | Odd/ even | Nature of address indicator |  |  |  |  |  |  |
| 2 | INN Ind. | Numbering plan Ind. |  |  | Spare |  |  |  |
| 3 | 2nd address signal |  |  |  | 1st address signal |  |  |  |
| · · · |  |  |  |  |  |  |  |  |
| n | Filler (if necessary) |  |  |  | nth address signal |  |  |  |

Called party number parameter field

The following codes are used in the subfields of the called party number parameter field:

a)	Odd/even indicator

0	even number of address signals
1	odd number of address signals

b)	Nature of address indicator

0000000	spare
0000001	subscriber number @
0000010	unknown @
0000011	national (significant) number
0000100	international number

spare

reserved for national use

1111111	spare

c)	Internal network number indicator (INN ind.)

0	routing to internal network number allowed
1	routing to internal network number not allowed

d)	Numbering plan indicator

000	spare
001	ISDN (Telephony) numbering plan (Recommendation E.164)
010	spare
011	Data numbering plan (Recommendation X.121) @
100	Telex numbering plan (Recommendation F.69) @
101	reserved for national use
110	reserved for national use
111	spare

e)	Address signal

0000	digit 0
0001	digit 1
0010	digit 2
0011	digit 3
0100	digit 4
0101	digit 5
0110	digit 6
0111	digit 7
1000	digit 8
1001	digit 9
1010	spare
1011	code 11
1100	code 12
1101	spare
1110	spare
1111	ST

The most significant address signal is sent first. Subsequent address signals are sent in successive 4-bit fields.

f)	Filler

In case of an odd number of address signals, the filler code 0000 is inserted after the last address signal.

## 3.10	Calling party number

The format of the calling party number parameter field is shown in Figure 11.

FIGURE  11/Q.763

|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | Odd/ even | Nature of address indicator |  |  |  |  |  |  |
| 2 | NI | Numbering plan Ind. |  |  | Present. Ind. |  | Screening |  |
| 3 | 2nd address signal |  |  |  | 1st address signal |  |  |  |
| · · · |  |  |  |  |  |  |  |  |
| n | Filler (if necessary) |  |  |  | nth address signal |  |  |  |
|  | NOTE – When the address presentation restricted indicator indicates address not available, octets 3 to n are omitted. |  |  |  |  |  |  |  |

Calling party number parameter field

The following codes are used in the calling party number parameter field:

a)	Odd/even indicator

See 3.9 a)

b)	Nature of address indicator

0000000	spare
0000001	subscriber number
0000010	unknown @
0000011	national (significant) number @
0000100	international number

spare

reserved for national use

1111111	spare

NOTE 1 – Other types of nature of address indications (e.g. transit exchange identity) are for further study.

c)	Calling party number incomplete indicator (NI) @

0	complete
1	incomplete

d)	Numbering plan indicator

See 3.9 d)

e)	Address presentation restricted (Pres. Restric.) indicator

00	presentation allowed
01	presentation restricted
10	address not available (Note 2) @
11	spare

NOTE 2 – When the address is unavailable, the subfields in items a), b), c) and d) are coded with 0’s.

f)	Screening indicator

00	reserved (Note 3)
01	user provided, verified and passed
10	reserved (Note 3)
11	network provided

NOTE 3 – Code 00 and 10 are reserved for “user provided, not verified” and “user provided, verified and failed” respectively. Codes 00 and 10 are for national use.

g)	Address signal

0000	digit 0
0001	digit 1
0010	digit 2
0011	digit 3
0100	digit 4
0101	digit 5
0110	digit 6
0111	digit 7
1000	digit 8
1001	digit 9
1010	spare
1111	code 11
1100	code 12

spare

h)	Filler

See 3.9 f).

## 3.11	Calling party’s category

The format of the calling party’s category parameter field is shown in Figure 12.

FIGURE  12/Q.763

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Calling party’s category |  |  |  |  |  |  |  |

Calling party’s category parameter field

The following codes are used in the calling party’s category parameter field:

00000000	calling party’s category unknown at this time @
00000001	operator, language French
00000010	operator, language English
00000011	operator, language German
00000100	operator, language Russian
00000101	operator, language Spanish

00001001	reserved (see Recommendation Q.104) (Note) @
00001010	ordinary calling subscriber
00001011	calling subscriber with priority
00001100	data call (voice band data)
00001101	test call
00001110	spare
00001111	payphone

spare

reserved for national use

11111111	spare

NOTE – In national networks code 00001001 may be used to indicate that the calling party is a national operator.

## 3.12	Cause indicators

The format of the cause indicators parameter field is shown in Figure 13.

|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | Ext. | Coding standard |  | Spare | Location |  |  |  |
| 2 | Ext. | Cause value |  |  |  |  |  |  |
| 3 |  |  |  |  |  |  |  |  |
| · · · | Diagnostic(s) (if any) |  |  |  |  |  |  |  |
| 3n |  |  |  |  |  |  |  |  |
|  | NOTE – Octet 3 to 3n may be omitted or repeated, 3 to 3n. |  |  |  |  |  |  |  |

FIGURE  13/Q.763

Cause indicators parameter field

The codes to be used in the subfields of the cause indicators parameter fields are defined in the Q.6XX-Series Recommendations.

## 3.13	Circuit group supervision message type indicator

The format of the circuit group supervision message type indicator parameter field is shown in Figure 14.

FIGURE  14/Q.763

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| H | G | F | E | D | C | B | A |

Circuit group supervision message type indicator parameter field

The following codes are used in the circuit group supervision message type indicator parameter field:

bits	B	A:	Type indicator
0	0	maintenance oriented
0	1	hardware failure oriented
1	0	reserved for national use (used in 1984 version)
1	1	spare

bits	C	H:	Spare

## 3.14	Circuit state indicator @

The format of the circuit state indicator parameter field is shown in Figure 15.

FIGURE  15/Q.763

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| H | G | F | E | D | C | B | A |
| H | G | F | E | D | C | B | A |

Circuit state indicator parameter field

The number of octets in the circuit state indicator parameter field is equal to the specified range + 1. Each circuit state indicator octet is associated with a circuit identification code such that octet n is associated with circuit identification code m  n –1, where m is the circuit identification code contained in the message.

The following codes are used in each circuit state indicator octet:

a)	for bits D C = 00

bits	B	A:	Maintenance blocking state
0	0	transient
0	1	spare
1	0	spare
1	1	unequipped

b)	for bits D C not equal to 00

bits	B	A:	Maintenance blocking state
0	0	no blocking (active)
0	1	locally blocked
1	0	remotely blocked
1	1	locally and remotely blocked

bits	D	C:	Call processing state
0	1	circuit incoming busy
1	0	circuit outgoing busy
1	1	idle

bits	F	E:	Hardware blocking state (Note)
0	0	no blocking (active)
0	1	locally blocked
1	0	remotely blocked
1	1	locally and remotely blocked

bits	H-G:	Spare

NOTE – If bits F E are not coded 0 0, bits D C must be coded 1 1.

## 3.15	Closed user group interlock code

The format of the closed user group interlock code parameter field is shown in Figure 16.

FIGURE 16/Q.763

|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1st NI digit |  |  |  | 2nd NI digit |  |  |  |
| 2 | 3 rd NI digit |  |  |  | 4nd NI digit |  |  |  |
| 3 |  |  |  |  |  |  |  |  |
| 4 |  | Binary code |  |  |  |  |  |  |

Closed user group interlock code parameter field

The following codes are used in the subfields of the closed user group interlock code parameter field:

a)	Network identity (NI) (octets 1 and 2)

Each digit is coded in the binary coded decimal representation from 0 to 9. If the first digit of this field is coded 0 or 9, the TCC (Telephony Country Code) follows in the second to fourth NI digits (the most significant TCC digit is in the 2nd NI digit). If the TCC is one or two digits long, the excess digit(s) is inserted with the code for ROA or network identification, if necessary. If octet 2 is not required, it is coded all zeros.

Coding of the first digit as 1 or 8 is excluded.

If the first digit is not 0, 9, 1 or 8 this field contains a DNIC (Data Network Identification Code) as defined in Recommendation X.121.

b)	Binary code (octets 3 and 4)

A code allocated to a closed user group administered by a particular ISDN or data network. Bit 8 of octet 3 is the most significant and bit 1 of octet 4 is the least significant.

## 3.16	Connected number

The format of the connected number parameter field corresponds to the format shown in Figure 17.

FIGURE  17/Q.763

|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | Odd/ even | Nature of address indicator |  |  |  |  |  |  |
| 2 | Spare | Numbering plan Ind. |  |  | Present. Restric. |  | Screening indicator |  |
| 3 | 2nd address signal |  |  |  | 1st address signal |  |  |  |
| · · · |  |  |  |  |  |  |  |  |
| n | Filler (if necessary) |  |  |  | nth address signal |  |  |  |
|  | NOTE – When the address presentation restricted indicator indicates address not available, octets 3 to n are omitted, and the screening indicator is set to 11 network provided. |  |  |  |  |  |  |  |

Connected number parameter field

The following codes are used in the subfields of the connected number parameter field:

a)	Odd/even indicator: see 3.9 a)

b)	Nature of address indicator: see 3.9 b)

c)	Numbering plan indicator: see 3.9 d)

d)	Address presentation restricted indicator:

00	presentation allowed
01	presentation restricted
10	address not available
11	spare

NOTE – When the address is unavailable, the subfields in items a), b), and c) are coded with 0’s.

e)	Screening indicator: see 3.10 f)

f)	Address signal: see 3.10 g)

g)	Filler: see 3.9 f).

## 3.17	Connection request (open)

The format of the connection request parameter field is shown in Figure 18.

The following codes are used in the subfields of the connection request parameter field:

a)	Local reference

A code indicating the local reference allocated by the signalling connection control part to the end-to-end connection.

b)	Point code

A code identifying the signalling point at which the connection request originated.

FIGURE  18/Q.763

|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 |  |  |  |  |  |  |  |  |
| 2 |  | Call identity |  |  |  |  |  |  |
| 3 |  |  |  |  |  |  |  |  |
| 4 |  |  | Point code |  |  |  |  |  |
| 5 | Spare |  |  |  |  |  |  |  |
| 6 | Clase de protocolo |  |  |  |  |  |  |  |
| 7 | Crédito |  |  |  |  |  |  |  |
|  | NOTE – Octets 6 and 7 may be omitted if protocol class requested is 2. |  |  |  |  |  |  |  |

Connection request parameter field

c)	Protocol class

A code identifying in pure binary representation, the protocol class requested for the end-to-end connection.

d)	Credit

A code identifying in pure binary representation the window size requested for the end-to-end connection.

## 3.18	Continuity indicators

The format of the continuity indicators parameter field is shown in Figure 19.

FIGURE  19/Q.763

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| H | G | F | E | D | C | B | A |

Continuity indicators parameter field

The following codes are used in the continuity indicators parameter field:

bit	A:		Continuity indicator
0		continuity check failed
1		continuity check successful

bits	H-B:	Spare

## 3.19	Echo control information

The format of the echo control information parameter field is shown in Figure 20.

FIGURE  20/Q.763

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| H | G | F | E | D | C | B | A |

Echo control information parameter field

a)	Outgoing half echo control device response indicator

bits	BA	00	No information
01	Outgoing half echo control device not included
10	Outgoing half echo control device included
11	Spare

b)	Incoming half echo control device response indicator

bits	DC	00	No information
01	Incoming half echo control device not included
10	Incoming half echo control device included
11	Spare

c)	Outgoing half echo control device request indicator

bits	FE	00	No information
01	Outgoing half echo control device activation request
10	Outgoing half echo control device deactivation request
11	Spare

d)	Incoming half echo control device request indicator

bits	HG	00	No information
01	Incoming half echo control device activation request
10	Incoming half echo control device deactivation request
11	Spare

## 3.20	End of optional parameters indicator

The last optional parameter field of a message is followed by the end of optional parameters indicator, which occupies a one octet field containing all zeros.

## 3.21	Event information

The format of the event information parameter field is shown in Figure 21.

FIGURE  21/Q.763

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| H | G | F | E | D | C | B | A |

Event information parameter field

The following codes are used in the event indicator parameter field:

bits	GFEDCBA:	Event indicator

0000000	spare
0000001	ALERTING
0000010	PROGRESS
0000011	in-band information or an appropriate pattern is now available
0000100	call forwarded on busy
0000101	call forwarded on no reply
0000110	call forwarded unconditional
	spare

bit	H:	Event presentation restricted indicator @
0	no indication
1	presentation restricted

## 3.22	Facility indicator

The format of the facility indicator parameter field is shown in Figure 22.

FIGURE  22/Q.763

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Facility indicators |  |  |  |  |  |  |  |

Facility indicator parameter field

The following codes are used in the facility indicator parameter field:

00000000	spare
00000001	spare
00000010	user-to-user service

spare

## 3.23	Forward call indicators

The format of the forward call indicators parameter field is shown in Figure 23.

FIGURE  23/Q.763

|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | H | G | F | E | D | C | B | A |
| 2 | P | O | N | M | L | K | J | I |

Forward call indicators parameter field

The following codes are used in the forward call indicators parameter field:

bit	A:	National/international call indicator
0	call to be treated as a national call
1	call to be treated as an international call

This bit can be set to any value in the country of origin. In the international network this bit is not checked. In the destination country, calls from the international network will have this bit set to 1.

bits	C	B:	End-to-end method indicator (Note)
0	0	no end-to-end method available (only link-by-link method available)
0	1	pass along method available @
1	0	SCCP method available 
1	1	pass along and SCCP methods available @

bit	D:	Interworking indicator (Note)
0	no interworking encountered (No. 7 signalling all the way)
1	interworking encountered

bit	E:	End-to-end information indicator (Note) @
0	no end-to-end information available
1	end-to-end information available

bit	F:	ISDN user part indicator (Note)
0	ISDN user part not used all the way
1	ISDN user part used all the way

bits	H	G:	ISDN user part preference indicator
0	0	ISDN user part preferred all the way
0	1	ISDN user part not required all the way
1	0	ISDN user part required all the way
1	1	spare

bit	I:		ISDN access indicator
0		originating access non-ISDN
1		originating access ISDN

bits	K	J:	SCCP method indicator
0	0	no indication
0	1	connectionless method available @
1	0	connection oriented method available
1	1	connectionless and connection oriented methods available @

bit	L:		Spare

bits	P-M:	Reserved for national use

NOTE – Bits B-F and J-K constitute the protocol control indicator.

## 3.24	Generic digits @

The format of the generic digits parameter field is shown in Figure 24.

FIGURE  24/Q.763

|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | Encoding sheme |  |  | Type of digits |  |  |  |  |
| 2 | digits |  |  |  |  |  |  |  |
| · |  |  |  |  |  |  |  |  |
| n | digits |  |  |  |  |  |  |  |

Generic digits parameter

The following codes are used in the subfields of the generic digits parameter:

a)	Encoding scheme

000	BCD even: (even number of digits)
001	BCD odd: (odd number of digits)
010	IA5 character
011	binary coded

spare

b)	Type of digits

00000	reserved for account code
00001	reserved for authorisation code
00010	reserved for private networking travelling class mark
00011	reserved for business communication group identity

spare for international use

spare for national use

11111	reserved for extension

c)	Digit

Coding in accordance to the coding scheme and type of digits.

## 3.25	Generic notification indicator

The format of the generic notification indicator parameter field is shown in Figure 25.

FIGURE  25/Q.763

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Ext. | Notification indicator |  |  |  |  |  |  |

Generic notification indicator parameter field

The following codes are used in the generic notification indicator parameter field:

a)	Extension indicator

0	information continues in the next octet
1	last octet

b)	Notification indicator

(used in DSS 1)

0000100	call completion delay

1000010	conference established
1000011	conference disconnected
1000100	other party added
1000101	isolated
1000110	reattached
1000111	other party isolated
1001000	other party reattached
1001001	other party split
1001010	other party disconnected
1001011	conference floating

1100000	call is a waiting call

1101000	diversion activated (used in DSS 1)
1101001	call transfer, alerting
1101010	call transfer, active

1111001	remote hold
1111010	remote retrieval
1111011	call is diverting

All other values are currently not used and are reserved for further extensions.

## 3.26	Generic number

The format of the generic number parameter field is shown in Figure 26.

FIGURE  26/Q.763

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Number qualifier indicator |  |  |  |  |  |  |  |
| Odd/ even | Nature of address indicator |  |  |  |  |  |  |
| NI Ind. | Numbering plan Ind. |  |  | Present. Ind. |  | Screening |  |
| 2nd address signal |  |  |  | 1st address signal |  |  |  |
| Filler (if necessary) |  |  |  | nth address signal |  |  |  |

Generic number parameter field

The following codes are used in the generic number parameter field:

a)	Number qualifier indicator

00000000	reserved (dialled digits) @
00000001	additional called number @
00000010	reserved (supplemental user provided calling number – failed network screening) @
00000011	reserved (supplemental user provided calling number – not screened) @
00000100	reserved (redirecting terminating number) @
00000101	additional connected number 
00000110	additional calling party number
00000111	additional original called number
00001000	additional redirecting number
00001001	additional redirection number

reserved (called freephone numbers) (spare)

reserved for national use

11111111	reserved for expansion

b)	Odd/even indicator: see 3.9 a)

c)	Nature of address indicator

0000000	spare
0000001	subscriber number
0000010	unknown
0000011	national (significant) number
0000100	international number

spare

reserved for national use

1111111	spare

NOTE 1 – For each supplementary service the relevant codes and possible default settings are described in the service description (see the Q.73xy-Series Recommendations).

d)	Number incomplete indicator

0	number complete
1	number incomplete

e)	Numbering plan indicator

000	spare
001	ISDN (telephony) numbering plan (Recommendation E.164)
010	spare 
011	data numbering plan (Recommendation X.121) @
100	telex numbering plan (Recommendation F.69) @
101	private numbering plan
110	reserved for national use
111	spare

NOTE 2 – For each supplementary service the relevant codes and possible default settings are described in the service description (see the Q.73xy-Series Recommendations).

f)	Address presentation restricted indicator

00	presentation allowed
01	presentation restricted
10	address not available
11	spare

NOTE 3 – For each supplementary service the relevant codes and possible default settings are described in the service description (see the Q.73xy-Series Recommendations).

g)	Screening indicator

Only used if the number qualifier indicator is coded 0000 0001 or 0000 0010 this indicator is coded as follows:

00	user provided, not verified
01	user provided, verified and passed
10	user provided, verified and failed
11	network provided

NOTE 4 – For each supplementary service the relevant codes and possible default settings are described in the service description (see the Q.73xy-Series Recommendations).

h)	Address signal

0000	digit 0
0001	digit 1
0010	digit 2
0011	digit 3
0100	digit 4
0101	digit 5
0110	digit 6
0111	digit 7
1000	digit 8
1001	digit 9

spare

i)	Filler: 3.9 f)

## 3.27	Generic reference (reserved)

The format of the generic reference parameter field is shown in Figure 27.

|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | Reference qualifier indicator |  |  |  |  |  |  |  |
| 2 | Length of reference |  |  |  |  |  |  |  |
| 3 | Reference 1st octet |  |  |  |  |  |  |  |

| n  3 | Reference nth octet |
| --- | --- |

FIGURE  27/Q.763

Generic reference parameter field

a)	Reference qualifier indicator

0000 0000	spare
0000 0001	CCBS context identifier (for further study)
0000 0010	reserved
0000 0011	reserved

spare

b)	Length of reference

0000 0000	spare
0000 0001	length of 1 octet
0000 0010	length of 2 octets
0000 0011	length of 3 octets
0000 0100	length of 4 octets

spare

c)	Reference nth octet

A code expressing in pure binary representation the reference number of the context given by the entity which handles and provides the service.

## 3.28	Information indicators @

The format of the information indicators parameter field is shown in Figure 28.

FIGURE  28/Q.763

|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | H | G | F | E | D | C | B | A |
| 2 | P | O | N | M | L | K | J | I |

Information indicators parameter field

The following codes are used in the information indicators parameter field:

bits	B	A:	Calling party address response indicator
0	0	calling party address not included
0	1	calling party address not available
1	0	spare
1	1	calling party address included

bit	C:		Hold provided indicator 
0		hold not provided
1		hold provided

bits	E-D:		Spare

bit	F:		Calling party’s category response indicator
0		calling party’s category not included
1		calling party’s category included

bit	G:		Charge information response indicator
0		charge information not included
1		charge information included

bit	H:		Solicited information indicator
0		solicited
1		unsolicited

bits	L-I:		Spare

bits	P-M:	Reserved

## 3.29	Information request indicators @

The format of the information request indicators parameter field is shown in Figure 29.

FIGURE  29/Q.763

|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | H | G | F | E | D | C | B | A |
| 2 | P | O | N | M | L | K | J | I |

Information request indicators parameter field

The following codes are used in the information request indicators parameter field:

bit	A:		Calling party address request indicator
0		calling party address not requested
1		calling party address requested

bit	B:		Holding indicator
0		holding not requested
1		holding requested

bit	C:		Spare

bit	D:		Calling party’s category request indicator
0		calling party’s category not requested
1		calling party’s category requested

bit	E:		Charge information request indicator
0		charge information not requested
1		charge information requested

bits	G-F:	Spare

bit	H:		Malicious call identification request indicator
0		malicious call identification not requested
1		reserved for malicious call identification requested

bits	L-I:	Spare

bits P-M:	Reserved

## 3.30	Location number

The format of the location number field is shown in Figure 30.

FIGURE  30/Q.763

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Odd/ even | Nature of address indicator |  |  |  |  |  |  |
| INN/ NI Ind. | Numbering plan Ind. |  |  | Present. Ind. |  | Screening |  |
| 2nd address signal |  |  |  | 1st address signal |  |  |  |
| Filler (if necessary) |  |  |  | nth address signal |  |  |  |

Location number parameter field

The following codes are used in the subfields of the location number parameter field:

a)	Odd/even indicator: see 3.9 a).

b)	Nature of address indicator

0000000	spare
0000001	reserved for subscriber number @
0000010	reserved for unknown @
0000011	national (significant) number @
0000100	international number

spare

reserved for national use

1111111	spare

c)	Internal network number indicator (INN)

0	routing to internal number allowed
1	routing to internal number not allowed

NOTE 1 – Default set to 1.

d)	Numbering plan indicator

000	spare
001	ISDN (telephony) numbering plan (Recommendation E.164)
010	spare
011	Data numbering plan (Recommendation X.121) @
100	Telex numbering plan (Recommendation F.69) @
101	private numbering plan
110	reserved for national use
111	spare

NOTE 2 – Default set to 001.

e)	Address presentation restricted (Pres. restric.) indicator

00	presentation allowed
01	presentation restricted
10	address not available (Note 3) @
11	spare

NOTE 3 – Default set to 01.

f)	Screening indicator

00	reserved
01	user provided, verified and passed
10	reserved
11	network provided

NOTE 4 – Default set to 11.

g)	Address signals: see 3.26 h)

h)	Filler: see 3.9 f)

## 3.31	MCID request indicators

The format of the MCID request indicators parameter field is shown in Figure 31.

FIGURE  31/Q.763

| H | G | F | E | D | C | B | A |
| --- | --- | --- | --- | --- | --- | --- | --- |
| MCID request indicators |  |  |  |  |  |  |  |

MCID request indicators parameter field

The following codes are used in the MCID request indicators parameter field:

bit 	A:		MCID request indicator
0		MCID not requested
1		MCID requested

bit 	B:		Holding indicator @
0		Holding not requested
1		Holding requested

bits H-C:	Spare

## 3.32	MCID response indicators

The format of the MCID response indicators parameter field is shown in Figure 32.

FIGURE  32/Q.763

| H | G | F | E | D | C | B | A |
| --- | --- | --- | --- | --- | --- | --- | --- |
| MCID response indicators |  |  |  |  |  |  |  |

MCID response indicators parameter field

The following codes are used in the MCID response indicators parameter field:

bit 	A:		MCID response indicator
0		MCID not included
1		MCID included

bit	B:		Hold provided indicator @
0		Holding not provided
1		Holding provided

bits H-C:	Spare

## 3.33	Message compatibility information

The format of the message compatibility information parameter field is shown in Figure 33.

FIGURE  33/Q.763

|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 2 | Instruction indicator |  |  |  |  |  |  |  |

Message compatibility information parameter field

The following codes are used in the subfields of the parameter compatibility information parameter field:

a)	Instruction indicators

The format of the instruction indicators subfield is shown in Figure 34.

FIGURE  34/Q.763

| 2 | Ext/1 | G | F | E | D | C | B | A |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 2a | Ext/1 | More instruction indicators if required |  |  |  |  |  |  |
|  | · · · |  |  |  |  |  |  |  |
| 2n | 1 | More instruction indicators if required |  |  |  |  |  |  |

Instruction indicators subfield

The following codes are used in the instructions indicators subfield:

bit	A:		Transit at intermediate exchange indicator
0		transit interpretation
1		end node interpretation

bit	B:		Release call indicator
0		do not release call
1		release call

bit	C:		Send notification indicator (Note 1)
0		do not send notification
1		send notification

NOTE 1 – Notification by confusion message.

bit	D:		Discard message indicator
0		do not discard message (pass on)  (Note 2)
1		discard message

bits	G-F:		Spare

NOTE 2 – If pass on is set (bit D = 0) but not possible then bits C and E are checked.

bit 	E:		Pass on not possible indicator
0		release call
1		discard information

b)	Extension indicator

0		next octet exists
1		last octet

c)	More instruction indicators

The bits will be defined when required.

## 3.34	MLPP precedence

The format of the MLPP precedence parameter field is shown in Figure 35.

FIGURE  35/Q.763

|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | Spare | LFB |  | Spare | Precedence level |  |  |  |
| 2 | 1st NI digit |  |  |  | 2nd NI digit |  |  |  |
| 3 | 3rd NI digit |  |  |  | 4th digit |  |  |  |
| 4 |  |  |  |  |  |  |  |  |
| 5 | MLPP service domain |  |  |  |  |  |  |  |
| 6 |  |  |  |  |  |  |  |  |

MLPP precedence parameter field

The following codes are used in the subfields of the MLPP precedence parameter field:

a)	LFB

00	LFB allowed
01	Path reserved @
10	LFB not allowed
11	spare

b)	Precedence level

0000	flash override
0001	flash
0010	immediate
0011	priority
0100	routine

spare

c)	Network identity NI octet 2 and 3

Each digit is coded in binary coded decimal representation from 0 to 9.

The first digit of this field is coded 0. The TCC (Telephony Country Code) follows in the second to fourth NI digits (the most significant TCC digit is in the 2nd NI digit). If the TCC is one or two digits long, the excess digit(s) is inserted with the code for ROA or network identification, if necessary. If octet 2 is not required, it is coded all zeros.

d)	MLPP service domain (octets 4, 5 and 6)

A code pure binary coded allocated to a MLPP service domain administered by a particular ISDN. Bit 8 of octet 4 is the most significant, and bit 1 of octet 6 is the least significant, respectively.

## 3.35	Nature of connection indicators

The format of the nature of connection indicators parameter field is shown in Figure 36.

FIGURE  36/Q.763

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| H | G | F | E | D | C | B | A |

Nature of connection indicators parameter field

The following codes are used in the nature of connection indicators parameter field:

bits	B	A:	Satellite indicator
0	0	no satellite circuit in the connection
0	1	one satellite circuit in the connection
1	0	two satellite circuits in the connection
1	1	spare

bits	D	C:	Continuity check indicator
0	0	continuity check not required
0	1	continuity check required on this circuit
1	0	continuity check performed on a previous circuit
1	1	spare

bit	E:		Echo control device indicator
0		outgoing half echo control device not included
1		outgoing half echo control device included

bits	F-H:	Spare

## 3.36	Network specific facility @

The format of the network specific facility parameter field is shown in Figure 37.

FIGURE  37/Q.763

|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Length of network identification |  |  |  |  |  |  |  |
| 1a | 1 Ext | Type of network id. |  |  | Network identification plan |  |  |  |
| 1b ·1 ·1 | 0 Spare | Network identification |  |  |  |  |  |  |
| 1m | 0 |  |  |  |  |  |  |  |
| 2 · n | Network specific facility |  |  |  |  |  |  |  |

Network specific facility parameter field

The following codes are used in the subfield of the network specific facility parameter field:

a)	Length of network identification

This field contains the length, in octets, of the network identification found in octets 1a, 1b-1m. If the value is 0000 0000 then octets 1a-1m are omitted.

b)	Type of network identification

The following codes are used in the type of network identification subfield

Bits	7 6 5
0 1 0		national network identification
0 1 1		reserved for international network identification
others	spare

When the type of network identification is coded 010 “national network identification”, the network identification plan and network identification are coded nationally.

c)	Network identification plan

d)	Network identification

e)	Network-specific facility

This field is encoded according to the rules specified by the identified network. The network may specify the same coding rule as stimulus type of information elements in Recommendation Q.932. In this case multiple information elements may be included in this field.

## 3.37	Optional backward call indicators

The format and codes of the optional backward call indicators field are shown in Figure 38.

FIGURE  38/Q.763

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| H | G | F | E | D | C | B | A |

Optional backward call indicators parameter field

The following codes are used in the optional backward call indicators parameter field:

bit	A:		In-band information indicator
0		no indication
1		in-band information or an appropriate pattern is now available

bit	B:		Call diversion may occur indicator
0		no indication
1		call diversion may occur

bit	C:		Simple segmentation indicator
0		no additional information will be sent
1		additional information will be sent in a segmentation message

bit	D:		MLPP user indicator
0		no indication
1		MLPP user

bits E-H:	Reserved for national use

## 3.38	Optional forward call indicators

The format of the optional forward call indicators parameter field is shown in Figure 39.

FIGURE  39/Q.763

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| H | G | F | E | D | C | B | A |

Optional forward call indicators parameter field

The following codes are used in the optional forward call indicators parameter field:

bits	B	A:	Closed user group call indicator
0	0	non-CUG call
0	1	spare
1	0	closed user group call, outgoing access allowed
1	1	closed user group call, outgoing access not allowed

bit	C:		Simple segmentation indicator
0		no additional information will be sent
1		additional information will be sent in a segmentation message

bits D-G:	Spare

bit	H:		Connected line identity request indicator
0		not requested
1		requested

## 3.39	Original called number

The format of the original called number parameter field corresponds to the format shown in Figure 40.

FIGURE  40/Q.763

|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | Odd/ even | Nature of address indicator |  |  |  |  |  |  |
| 2 | Spare | Numbering plan Ind. |  |  | Present. Ind. |  | Spare |  |
| 3 | 2nd address signal |  |  |  | 1st address signal |  |  |  |
| · · · |  |  |  |  |  |  |  |  |
| n | Filler (if necessary) |  |  |  | nth address signal |  |  |  |

Original called number parameter field

The following codes are used in the subfields of the original called number parameter field:

a)	Odd/even indicator: see 3.9 a)

b)	Nature of address indicator: see 3.9 b)

c)	Numbering plan indicator: see 3.9 d)

d)	Address presentation restricted indicator: see 3.10 e)

e)	Address signal: see 3.10 g)

f)	Filler: see 3.9 f).

## 3.40	Origination ISC point code

The format of the origination ISC point code parameter field is shown in Figure 50.

## 3.41	Parameter compatibility information

The format of the parameter compatibility information parameter field is shown in Figure 41.

FIGURE  41/Q.763

|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1st upgraded parameter |  |  |  |  |  |  |  |
| 2 | Instruction indicators |  |  |  |  |  |  |  |
|  | · · · |  |  |  |  |  |  |  |
| n | nth upgraded parameter |  |  |  |  |  |  |  |
| n1 | Instruction indicators |  |  |  |  |  |  |  |

Parameter compatibility information parameter field

The following codes are used in the subfields of the parameter compatibility information parameter field.

a)	Nth upgraded parameter name

This field contains the parameter name of the nth upgraded parameter in accordance with Table 5.

b)	Instruction indicators

The format of the instruction indicators subfield is shown in Figure 34.

The following codes are used in the instructions indicators subfield:

bit 	A:		Transit at intermediate exchange indicator
0		transit interpretation
1		end node interpretation

bit 	B:		Release call indicator
0		do not release call
1		release call

bit 	C:		Send notification indicator (Note 1)
0		do not send notification
1		send notification

NOTE 1 – Notification by confusion or a release complete message.

bit 	D:		Discard message indicator
0		do not discard message (pass on)  (Note 2)
1		discard message

NOTE 2 – If pass on is set (bit D = 0) but not possible then bits C, F and G are checked.

bit	E:		Discard parameter indicator
0		do not discard parameter (pass on) (Note 3)
1		discard parameter

NOTE 3 – If pass on is set (bit E = 0) but not possible then bits C, F and G are checked.

bits G-F:		Spare

c)	Extension indicator

0	next octet exists
1	last octet

d)	More instruction indicators

The bits will be defined when required.

## 3.42	Propagation delay counter

The format of the propagation delay counter parameter field is shown in Figure 42.

FIGURE  42/Q.763

|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 2 | Propagation delay value |  |  |  |  |  |  |  |

Propagation delay counter parameter field

The propagation delay counter parameter expresses in pure binary representation the propagation delay value of a call in ms to be accumulated during call set-up.

## 3.43	Range and status

The format of the range and status parameter field is shown in Figure 43.

|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | Range |  |  |  |  |  |  |  |  |
| 2 |  |  |  |  |  |  |  |  |  |
| . |  |  |  |  |  |  |  |  |  |
| . | Status |  |  |  |  |  |  |  |  |
| . |  |  |  |  |  |  |  |  |  |
| n |  |  |  |  |  |  |  |  |  |

FIGURE  43/Q.763

Range and status parameter field

Recommendation Q.763     (03/93)

The following codes are used in the subfields of the range and status parameter field:

a)	Range

A number in pure binary representation ranging from 1 to 255. The number represented by the range code +1 indicates the range of circuits affected by the message.

b)	Status

The status subfield contains from 2 to 256 status bits numbered from 0 to 255. Status bit 0 is located in bit position 1 of the first status subfield octet. Other status bits follow in numerical order. The number of relevant status bits in a given status subfield is equal to range +1.

Each status bit is associated with a circuit identification code such that status bit n is associated with circuit identification code m + n, where m is the circuit identification code contained in the message.

The status bits are coded as follows:

–	in circuit group blocking messages

0	no indication
1	blocking

–	in circuit group blocking acknowledgement messages

0	no indication
1	blocking acknowledgement

–	in circuit group unblocking messages

0	no indication
1	unblocking

–	in circuit group unblocking acknowledgement messages

0	no indication
1	unblocking acknowledgement

–	in circuit group reset acknowledgement messages

0	not blocked for maintenance reasons
1	blocked for maintenance reasons

The number of circuits affected by a group supervision message is limited to 32 or less. For the group reset messages this requires that the range value be 31 or less. For the group blocking and unblocking messages the range value may be up to 255, but the number of status bits set to 1 must be 32 or less.

For the group blocking, unblocking and reset messages, range code 0 is reserved.

## 3.44	Redirecting number

The format of the redirecting number parameter field corresponds to the format shown in Figure 40.

The following codes are used in the subfields of the redirecting number parameter field:

a)	Odd/even indicator:  see 3.9 a)

b)	Nature of address indicator:  see 3.9 b)

c)	Numbering plan indicator:  see 3.9 d)

d)	Address presentation restricted indicator:  see 3.10 e)

e)	Address signal:  see 3.10 g)

f)	Filler:  see 3.9 f).

## 3.45	Redirection information

The format of the redirection information parameter field is shown in Figure 44.

FIGURE  44/Q.763

|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | H | G | F | E | D | C | B | A |  |
| 2 | P | O | N | M | L | K | J | I |  |
| NOTE – Octet 2 is omitted if the redirection counter is coded 001. |  |  |  |  |  |  |  |  |  |

Redirection information parameter field

The following codes are used in the redirection information parameter field:

bits	C	B	A:	Redirecting indicator

0	0	0	no redirection @

0	0	1	call rerouted @

0	1	0	call rerouted, all redirection information presentation restricted @

0	1	1	call diversion

1	0	0	call diversion, all redirection information presentation restricted

1	0	1	call rerouted, redirection number presentation restricted @

1	1	0	call diversion, redirection number presentation restricted

1	1	1	spare

bit	D:	Spare

bits	H	G	F	E:	Original redirection reasons

0	0	0	0	unknown/not available

0	0	0	1	user busy @

0	0	1	0	no reply @

0	0	1	1	unconditional @

0	1	0	0  

a		0  	spare

1	1	1	1  

bits	K	J	I:		Redirection counter. Number of redirections the call has undergone expressed as a binary number between 1 and 5.

bit	L:	Spare

bits	P	O	N	M:	Redirecting reason

0	0	0	0	unknown/not available

0	0	0	1	user busy

0	0	1	0	no reply

0	0	1	1	unconditional

0	1	0	0	deflection during alerting

0	1	0	1	deflection immediate response

0	1	1	0	mobile subscriber not reachable

0	1	1	1  

a		0  	spare

1	1	1	1  

## 3.46	Redirection number

The format of the redirection number parameter field corresponds to the format shown in Figure 10.

The following codes are used in the subfields of the redirection number parameter field:

a)	Odd/even indicator:  see 3.9 a)

b)	Nature of address indicator:  see 3.9 b)

c)	Internal network number indicator:  see 3.9 c)

d)	Numbering plan indicator:  see 3.9 d)

e)	Address signal:  see 3.10 g)

f)	Filler:  see 3.9 f).

## 3.47	Redirection number restriction parameter

The format of the redirection number presentation parameter field is shown in Figure 45.

FIGURE  45/Q.763

|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | H | G | F | E | D | C | B | A |  |

Redirection number restriction parameter field

The following codes are used in the redirection number restriction number parameter field:

bits	B	A:	Presentation restricted indicator
0	0	Presentation allowed
0	1	Presentation restricted
1	0	spare
1	1	spare

bits	H-C:	Spare

## 3.49	Service activation @

The format of the service activation parameter field is shown in Figure 49.

|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | Feature code 1 |  |  |  |  |  |  |  |  |
| 2 | Feature code 2 |  |  |  |  |  |  |  |  |
| 3 | Feature code 3 |  |  |  |  |  |  |  |  |
| n | Feature code n |  |  |  |  |  |  |  |  |

FIGURE  49/Q.763

Service activation parameter field

The following feature codes are used in the service activation parameter field:

0000 0000	

0000to		reserved for international use

0111 1011	

0111 1100	

0000to		reserved for international use

1111 1110	

1111 1111	reserved for extension

## 3.50	Signalling point code @

The format of the signalling point code parameter field is shown in Figure 50.

|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | Signalling point code |  |  |  |  |  |  |  |  |
| 2 | Spare |  |  |  |  |  |  |  |  |

FIGURE  50/Q.763

Signalling point code parameter field

## 3.51	Subsequent number

The format of the subsequent number parameter field is shown in Figure 51.

|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | Odd/ even | Spare |  |  |  |  |  |  |  |
| 2 | 2nd address signal |  |  |  | 1st address signal |  |  |  |  |
| . . . |  |  |  |  |  |  |  |  |  |
| n | Filler (if necessary) |  |  |  | nth address signal |  |  |  |  |

FIGURE  51/Q.763

Subsequent number parameter field

The following codes are used in the subfields of the subsequent number parameter field:

a)	Odd/even indicator:  see 3.9 a)

b)	Address signal:  see 3.9 e)

c)	Filler:  see 3.9 f).

## 3.52	Suspend/resume indicators

The format of the suspend/resume indicators parameter field is shown in Figure 52.

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| H | G | F | E | D | C | B | A |

FIGURE  52/Q.763

Suspend/resume indicators parameter field

The following codes are used in the suspend/resume indicators parameter field:

bit	A:	Suspend/resume indicator

0	ISDN subscriber initiated

1	network initiated

bits	B-H:	Spare

## 3.53	Transit network selection @

The format of the transit network selection parameter field is shown in Figure 53.

|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Odd/ even | Type of network identification |  |  | Network identification plan |  |  |  |  |
| 2 |  |  |  |  |  |  |  |  |  |
|  | Network identification |  |  |  |  |  |  |  |  |
| n |  |  |  |  |  |  |  |  |  |

FIGURE  53/Q.763

Transit network selection parameter field

The following codes are used in the subfields of the transit network selection parameter field:

a)	Odd/even indicator

0		even number of digits
1		odd number of digits

b)	Type of network identification

000	CCITT-standardized identification
010	national network identification
other	spare

c)	Network identification plan

i)	For CCITT-standardized identification

0000	unknown

0011	public data network identification code (DNIC), Recommendation X.121

0110	public land mobile network identification code (MNIC), Recommendation E.212

other	spare

ii)	For national network identification

This information is coded according to national specifications.

d)	Network identification

This information is organized according to the network identification plan and the coding principle given in 3.9 e).

## 3.54	Transmission medium requirement

The format of the transmission medium requirement parameter field is shown in Figure 54.

The following codes are used in the transmission medium requirement parameter field:

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Transmission medium requirement (TMR) |  |  |  |  |  |  |  |

FIGURE  54/Q.763

Transmission medium requirement parameter field

00000000	speech
00000001	spare
00000010	64 kbit/s unrestricted
00000011	3.1 kHz audio
00000100	reserved for alternate speech (service 2)/64 kbit/s unrestricted (service 1) 
00000101	reserved for alternate 64 kbit/s unrestricted (service 1)/speech (service 2) 
00000110	64 kbit/s preferred
00000111	2 × 64 kbit/s unrestricted
00001000	384 kbit/s unrestricted
00001001	1536 kbit/s unrestricted

00001010	1920 kbit/s unrestricted

00001011	

000to		spare

11111111	

## 3.55	Transmission medium requirement prime

The format of the transmission medium requirement prime parameter field corresponds to the format shown in Figure 54, except that the coding rules for optional parameter are applied.

The following codes are used in the transmission medium requirement prime parameter field:

00000000	speech
00000001	spare
00000010	reserved for 64 kbit/s unrestricted 
00000011	3.1 kHz audio
00000100	reserved for alternate speech (service 2)/64 kbit/s unrestricted (service 1)
00000101	reserved for alternate 64 kbit/s unrestricted (service 1)/speech (service 2)
00000110	reserved for 64 kbit/s preferred
00000111	reserved for 2 × 64 kbit/s unrestricted
00001000	reserved for 384 kbit/s unrestricted
00001001	reserved for 1536 kbit/s unrestricted
00001010	reserved for 1920 kbit/s unrestricted

00001011	

000to		spare

11111111	

## 3.56	Transmission medium used

The format of the transmission medium used parameter field corresponds to the format shown in Figure 54, except that the coding rules for optional parameter are applied.

The codings are identical to codings in section 3.55.

## 3.57	User service information

The format of the user service information parameter field is shown in Figure 55. This format is the same as the bearer capability information element from Recommendation Q.931 and not all capabilities coded here are supported at this time.

The codes to be used in the subfields of the user service information parameter field are defined in the bearer capability information element in Recommendation Q.931.

FIGURE  55/Q.763

|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | Ext. | Coding standard |  | Information transfer capability |  |  |  |  |
| 2 | Ext. | Transfer mode |  | Information transfer rate |  |  |  |  |
| 2a | Ext. | Structure |  |  | Configuration |  | Establishment |  |
| 2b | Ext. | Symmetry |  | Iinformation transfer rate (destination to origination) |  |  |  |  |
| 3 | Ext. | Layer ident. |  | User information layer 1 protocol |  |  |  |  |
| 4 | Ext. | Layer ident. |  | User information layer 2 protocol |  |  |  |  |
| 5 | Ext. | Layer ident. |  | User information layer 3 protocol |  |  |  |  |
| NOTES 1 Octet 2a is omitted if default values apply to all fields of octets 2a and 2b. 2 Octet 2b is omitted if default values apply to all fields of octet 2b. 3 Octets 3, 4, 5 or any combination of these octets may be omitted. Octet 3 may be extended as described in Table 4-6/Q.931. |  |  |  |  |  |  |  |  |

User service information parameter field

## 3.58	User service information prime

The format of the user service information prime parameter field is shown in Figure 55.

The codes used for the user service information prime parameter field correspond to the codes shown in clause 4/Q.931.

## 3.59	User teleservice information

The format of the user teleservice information parameter field is shown in Figure 56. This format is the same as the high layer compatibility information element from Recommendation Q.931 and not all capabilities coded here are supported at this time.

The codes to be used in the user teleservice information parameter field are defined in the high layer compatibility information element in Recommendation Q.931.

|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 |  | Coding standard |  | Interpretation |  |  | Presentation |  |  |
| 2 | 0/1 ext. | High layer characteristics identification |  |  |  |  |  |  |  |
|  | 1 ext. | Extended high layer characteristics identification |  |  |  |  |  |  |  |

FIGURE  56/Q.763

User teleservice information parameter field

## 3.60	User-to-user indicators

The format of the user-to-user indicators parameter field is shown in Figure 57.

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| H | G | F | E | D | C | B | A |

FIGURE  57/Q.763

User-to-user indicators parameter field

The following codes are used in the user-to-user indicators parameter field:

bit	A:		Type

0		request
1		response

If bit A equals 0 (request):

bits	C	B:	Service 1
0	0	no information
0	1	spare
1	0	request, not essential
1	1	request, essential

bits	E	D:	Service 2
0	0	no information
0	1	spare
1	0	request, not essential
1	1	request, essential

bits	G	F:	Service 3
0	0	no information
0	1	spare
1	0	request, not essential
1	1	request, essential

bit	H:		Spare

If bit A equals 1 (response):

bits	C	B:	Service 1
0	0	no information
0	1	not provided
1	0	provided
1	1	spare

bits	E	D:	Service 2
0	0	no information
0	1	not provided
1	0	provided
1	1	spare

bits	G	F:	Service 3
0	0	no information
0	1	not provided
1	0	provided
1	1	spare

bit	H:		Network discard indicator

## 0		no information
1		user-to-user information discarded by the network

## 3.61	User-to-user information

The format of the user-to-user information parameter is shown in Figure 58.

The format of the user-to-user information parameter field is coded identically to the protocol discriminator plus user information field described in 4.5.29/Q.931.

|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 |  |  |  |  |  |  |  |  |  |
| . |  |  |  |  |  |  |  |  |  |
| . | User-to-user information |  |  |  |  |  |  |  |  |
| . |  |  |  |  |  |  |  |  |  |
| n |  |  |  |  |  |  |  |  |  |

FIGURE  58/Q.763

User-to-user information parameter field

# 4	ISDN user part messages and codes

In Tables 21 to 49, the format and coding of ISDN user part messages is specified. For each message, a list of the relevant parameters is given and for each parameter:

–	a reference to the subclause where the formatting and coding of the parameter content is specified;

–	the type of the parameter. The following types are used in the tables:

F  =  mandatory fixed length parameter;

V  =  mandatory variable length parameter;

O  =  optional parameter of fixed or variable length;

–	the length of the parameter.

The value in the table includes:

–	for type F parameters the length, in octets, of the parameter content;

–	for type V parameters the length, in octets, of the length indicator and of the parameter content. The minimum and the maximum length are indicated;

–	for type O parameters the length, in octets, of the parameter name, length indicator and parameter content. For variable length parameters the minimum and maximum length is indicated.

For each message type, type F parameters and the pointers for the type V parameters must be sent in the order specified in Tables 21 to 49.

The routing label and circuit identification code fields, which are transmitted ahead of the message type field if required are not shown. Parameter names, pointers to mandatory variable fields and the optional part, and length indicators appear in the message in accordance with Figure 5 and are not shown explicitly in Tables 4 to 42.

TABLE  21/Q.763

Message Type:  Address complete

| TABLE 22/Q.763 Parameter | Reference (subclause) | Type | Length (octets) |
| --- | --- | --- | --- |
| Message type | 2.1 | F | 1 |
| Backward call indicators | 3.5 | F | 2 |
| Optional backward call indicators | 3.37 | O | 3 |
| Call reference @ | 3.8 | O | 7 |
| Cause indicators | 3.12 | O | 4-? |
| User-to-user indicators | 3.60 | O | 3 |
| User-to-user information | 3.61 | O | 3-131 |
| Access transport | 3.3 | O | 3-? |
| Generic notification ind. (Note) | 3.25 | O | 3 |
| Transmission medium used | 3.56 | O | 3 |
| Echo control information | 3.19 | O | 3 |
| Access delivery information | 3.2 | O | 3 |
| Redirection number | 3.46 | O | 5-12 |
| Parameter compatibility information | 3.41 | O | 4-? |
| Call diversion information | 3.6 | O | 3 |
| Network specific facility @ | 3.36 | O | 4-? |
| Remote operations @ | 3.48 | O | 3-? |
| Service activation @ | 3.49 | O | 3-? |
| Redirection number restr. | 3.47 | O | 3 |
| End of optional parameters | 3.20 | O | 1 |
| NOTE – This parameter may be repeated. |  |  |  |

Message Type:  Answer

| Parameter | Reference (subclause) | Type | Length (octets) |
| --- | --- | --- | --- |
| Message type | 2.1 | F | 1 |
| Backward call indicators | 3.5 | O | 4 |
| Optional backward call indicators | 3.37 | O | 3 |
| Call reference @ | 3.8 | O | 7 |
| User-to-user indicators | 3.60 | O | 3 |
| User-to-user information | 3.61 | O | 3-131 |
| Connected number | 3.16 | O | 4-12 |
| Access transport | 3.3 | O | 3-? |
| Access delivery information | 3.2 | O | 3 |
| Generic notification ind. (Note) | 3.25 | O | 3 |
| Parameter compatibility information | 3.41 | O | 4-? |
| Call history information | 3.7 | O | 4 |
| Generic number (Note) | 3.26 | O | 4-12 |
| Transmission medium used | 3.56 | O | 3 |
| Network specific facility @ | 3.36 | O | 4-? |
| Remote operations @ | 3.48 | O | ? |
| Redirection number | 3.46 | O | 5-12 |
| Service activation @ | 3.49 | O | 3-? |
| Echo control information | 3.19 | O | 3 |
| Redirection number restr. | 3.47 | O | 3 |
| End of optional parameters | 3.20 | O | 1 |
| NOTE – This parameter may be repeated. |  |  |  |

TABLE  23/Q.763

Message Type:  Call progress

| Parameter | Reference (subclause) | Type | Length (octets) |
| --- | --- | --- | --- |
| Message type | 2.1 | F | 1 |
| Event information | 3.21 | F | 1 |
| Cause indicators | 3.12 | O | 4-? |
| Call reference @ | 3.8 | O | 7 |
| Backward call indicators | 3.5 | O | 4 |
| Optional backward call indicators | 3.37 | O | 3 |
| Access transport | 3.3 | O | 3-? |
| User-to-user indicators | 3.60 | O | 3 |
| Redirection number | 3.46 | O | 5-12 |
| User-to-user information | 3.61 | O | 3-131 |
| Generic notification ind. (Note) | 3.25 | O | 3 |
| Network specific facility @ | 3.36 | O | 4-? |
| Remote operations @ | 3.48 | O | ? |
| Transmission medium used | 3.56 | O | 3 |
| Access delivery information | 3.2 | O | 3 |
| Parameter compatibility information | 3.41 | O | 4 |
| Call diversion information | 3.6 | O | 3 |
| Service activation @ | 3.49 | O | 3-? |
| Redirection number restr. | 3.47 | O | 3 |
| End of optional parameters | 3.20 | O | 1 |
| NOTE – This parameter may be repeated. |  |  |  |

TABLE  24/Q.763

Message Type:  Circuit group query response @

| Parameter | Reference (subclause) | Type | Length (octets) |
| --- | --- | --- | --- |
| Message type | 2.1 | F | 1 |
| Range and statusa) | 3.43 | V | 2 |
| Circuit state indicator @ | 3.14 | V | 3-33 |
| a) The status subfield is not present. |  |  |  |

TABLE  25/Q.763

Message Type:  Circuit group reset acknowledgement

| Parameter | Reference (subclause) | Type | Length (octets) |
| --- | --- | --- | --- |
| Message type | 2.1 | F | 1 |
| Range and status | 3.43 | V | 3-34 |

TABLE  26/Q.763

Message Type:  Confusion

| Parameter | Reference (subclause) | Type | Length (octets) |
| --- | --- | --- | --- |
| Message type | 2.1 | F | 1 |
| Cause indicators | 3.12 | V | 4-20 |
| End of optional parameters | 3.20 | O | 1 |

TABLE  27/Q.763

Message Type:  Connect

| Parameter | Reference (subclause) | Type | Length (octets) |
| --- | --- | --- | --- |
| Message type | 2.1 | F | 1 |
| Backward call indicators | 3.5 | F | 2 |
| Optional backward call indicators | 3.37 | O | 3 |
| Connected number | 3.16 | O | 4-12 |
| Call reference @ | 3.8 | O | 7 |
| User-to-user indicators | 3.60 | O | 3 |
| User-to-user information | 3.61 | O | 3-131 |
| Access transport | 3.3 | O | 3-? |
| Network specific facility @ | 3.36 | O | 4-? |
| Generic notification ind. (Note) | 3.25 | O | 3 |
| Remote operations @ | 3.48 | O | ? |
| Transmission medium used | 3.56 | O | 3 |
| Echo control information | 3.19 | O | 3 |
| Access delivery information | 3.2 | O | 3 |
| Call history information | 3.7 | O | 3 |
| Parameter compatibility information | 3.41 | O | 4-? |
| Redirection number | 3.46 | O | 5-12 |
| Service activation @ | 3.49 | O | 3-? |
| Generic number (Note) | 3.26 | O | 4-12 |
| Redirection number restr. | 3.47 | O | 3 |
| End of optional parameters | 3.20 | O | 1 |
| NOTE – This parameter may be repeated. |  |  |  |

TABLE  28/Q.763

Message type:  Continuity

| Parameter | Reference (subclause) | Type | Length (octets) |
| --- | --- | --- | --- |
| Message type | 2.18 | F | 1 |
| Continuity indicators | 3.18 | F | 1 |

TABLE  29/Q.763

Message Type:  Facility reject

| Parameter | Reference (subclause) | Type | Length (octets) |
| --- | --- | --- | --- |
| Message type | 2.13 | F | 1 |
| Facility indicator | 3.22 | F | 1 |
| Cause indicators | 3.12 | V | 4 |
| User-to-user indicators | 3.60 | O | 3 |
| End of optional parameters | 3.20 | O | 1 |

TABLE  30/Q.763

Message Type:  Information  @

| Parameter | Reference (subclause) | Type | Length (octets) |
| --- | --- | --- | --- |
| Message type | 2.1 | F | 1 |
| Information indicators | 3.28 | F | 2 |
| Calling party’s category | 3.11 | O | 3 |
| Calling party number | 3.10 | O | 5-12 |
| Call reference | 3.8 | O | 7 |
| Connection request | 3.17 | O | 7-9 |
| Parameter compatibility information | 3.41 | O | 4-? |
| Network specific facility | 3.36 | O | 6-? |
| End of optional parameters | 3.20 | O | 1 |

TABLE  31/Q.763

Message Type:  Information request @

| Parameter | Reference (subclause) | Type | Length (octets) |
| --- | --- | --- | --- |
| Message type | 2.1 | F | 1 |
| Information request indicators | 3.29 | F | 2 |
| Call reference | 3.8 | O | 7 |
| Network specific facility | 3.36 | O | 4-? |
| Parameter compatibility information | 3.41 | O | 4-? |
| End of optional parameters | 3.20 | O | 1 |

TABLE  32/Q.763

Message Type:  Initial address

| Parameter | Reference (subclause) | Type | Length (octets) |
| --- | --- | --- | --- |
| Message type | 2.1 | F | 1 |
| Nature of connection indicators | 3.35 | F | 1 |
| Forward call indicators | 3.23 | F | 2 |
| Calling party’s category | 3.11 | F | 1 |
| Transmission medium requirement | 3.54 | F | 1 |
| Called party number | 3.9 | V | 4-11 |
| Transit network selection @ | 3.53 | O | 4-? |
| Call reference @ | 3.8 | O | 7 |
| Calling party number | 3.10 | O | 4-12 |
| Optional forward call indicators | 3.38 | O | 3 |
| Redirecting number | 3.44 | O | 4-12 |
| Redirection information | 3.45 | O | 3-4 |
| Closed user group interlock code | 3.15 | O | 6 |
| Connection request | 3.17 | O | 7-9 |
| Original called number | 3.39 | O | 4-12 |
| User-to-user information | 3.61 | O | 3-131 |
| Access transport | 3.3 | O | 3-? |
| User service information | 3.57 | O | 4-13 |
| User-to-user indicators | 3.60 | O | 3 |
| Generic number (Note 2) | 3.26 | O | 5-13 |
| Propagation delay counter | 3.42 | O | 4 |
| User service information prime | 3.58 | O | 4-13 |
| Network specific facility @ | 3.36 | O | 4-? |
| Generic digit @ (Note 1) | 3.24 | O | ? |
| Originating ISC point code | 3.40 | O | 4 |
| User service information prime | 3.58 | O | 7 |
| Remote operations @ | 3.48 | O | ? |
| Parameter compatibility information | 3.41 | O | 4-? |
| Generic notification (Note 1) | 3.25 | O | 3 |
| Service activation @ | 3.49 | O | 3-? |
| Generic reference (Note 2) | 3.27 | O | 5-? |
| MLPP precedence | 3.34 | O | 8 |
| Transmission medium requirement prime | 3.55 | O | 3 |
| Location number | 3.30 | O | 5-12 |
| End of optional parameters | 3.20 | O | 1 |
| NOTES 1 This parameter may be repeated. 2 For further study. |  |  |  |

TABLE  33/Q.763

Message Type:  Release

| Parameter | Reference (subclause) | Type | Length (octets) |
| --- | --- | --- | --- |
| Message type | 2.1 | F | 1 |
| Cause indicators | 3.12 | V | 3-? |
| Redirection information @ | 3.45 | O | 3-4 |
| Redirection number @ | 3.46 | O | 5-12 |
| Access transport | 3.3 | O | 3-? |
| Signalling point code @ | 3.50 | O | 4 |
| User-to-user information | 3.61 | O | 3-131 |
| Automatic congestion level | 3.4 | O | 4 |
| Network specific facility @ | 3.36 | O | 4-? |
| Access delivery information | 3.2 | O | 3 |
| Parameter compatibility information | 3.41 | O | 4-? |
| Redirection number restric. | 3.47 | O | 3 |
| User-to-user indicators | 3.60 | O | 3 |
| End of optional parameters | 3.20 | O | 1 |

TABLE  34/Q.763

Message Type:  Release complete

| Parameter | Reference (subclause) | Type | Length (octets) |
| --- | --- | --- | --- |
| Message type | 2.13 | F | 1 |
| Cause indicators | 3.12 | O | 5-6 |
| End of optional parameters | 3.20 | O | 1 |

TABLE  35/Q.763

Message Type:  Subsequent address

| Parameter | Reference (subclause) | Type | Length (octets) |
| --- | --- | --- | --- |
| Message type | 2.1 | F | 1 |
| Subsequent number | 3.51 | V | 3-10 |
| End of optional parameters | 3.20 | O | 1 |
| NOTE – No new optional parameters are allowed in the subsequent message. |  |  |  |

TABLE  36/Q.763

Message Type:  User-to-user information

| Parameter | Reference (subclause) | Type | Length (octets) |
| --- | --- | --- | --- |
| Message type | 2.1 | F | 1 |
| User-to-user information | 3.61 | V | 2-130 |
| Access transport | 3.3 | O | 3-? |
| End of optional parameters | 3.20 | O | 1 |

TABLE  37/Q.763

Message Type:  Forward transfer

| Parameter | Reference (subclause) | Type | Length (octets) |
| --- | --- | --- | --- |
| Message type | 2.1 | F | 1 |
| Call reference @ | 3.8 | O | 7 |
| End of optional parameters | 3.20 | O | 1 |

TABLE  38/Q.763

Message Type:  Resume, Suspend

| Parameter | Reference (subclause) | Type | Length (octets) |
| --- | --- | --- | --- |
| Message type | 2.1 | F | 1 |
| Suspend/resume indicators | 3.52 | F | 1 |
| Call reference @ | 3.8 | O | 7 |
| End of optional parameters | 3.20 | O | 1 |

TABLE  39/Q.763

Message Type:  Blocking

Message Type:  Blocking acknowledgement

Message Type:  Continuity check request

Message Type:  Loop back acknowledgement @

Message Type:  Overload @

Message Type:  Reset circuit

Message Type:  Unblocking

Message Type:  Unblocking acknowledgement

Message Type:  Unequipped circuit identification code @

| Parameter | Reference (subclause) | Type | Length (octets) |
| --- | --- | --- | --- |
| Message type | 2.1 | F | 1 |

TABLE  40/Q.763

Message Type:  Circuit group blocking

Message Type:  Circuit group blocking acknowledgement

Message Type:  Circuit group unblocking

Message Type:  Circuit group unblocking acknowledgement

| Parameter | Reference (subclause) | Type | Length (octets) |
| --- | --- | --- | --- |
| Message type | 2.13 | F | 1 |
| Circuit group supervision message type indicator | 3.13 | F | 1 |
| Range and status | 3.43 | V | 3-34 |

TABLE  41/Q.763

Message Type:  Circuit group reset

Message Type:  Circuit group query @

| Parameter | Reference (subclause) | Type | Length (octets) |
| --- | --- | --- | --- |
| Message type | 2.13 | F | 1 |
| Range and statusa) | 3.43 | V | 2 |
| a) The status subfield is not present. |  |  |  |

TABLE  42/Q.763

Message Type:  Facility accepted

Message Type:  Facility request

| Parameter | Reference (subclause) | Type | Length (octets) |
| --- | --- | --- | --- |
| Message type | 2.13 | F | 1 |
| Facility indicator | 3.22 | F | 1 |
| User-to-user indicators | 3.60 | O | 3 |
| Call reference @ | 3.83 | O | 7 |
| Connection request | 3.17 | O | 9 |
| Parameter compatibility information | 3.41 | O | 4-? |
| End of optional parameters | 3.20 | O | 1 |

TABLE  43/Q.763

Message Type:  Pass-along @

| Parameter | Reference (subclause) | Type | Length (octets) |
| --- | --- | --- | --- |
| Message type | 2.1 | F | 1 |
| Message type Mandatory fixed part Mandatory variable part Optional part | Any message in Tables 21 to 49 which is relevant only at the “endpoint” of a connection as defined in 3/Q.764 |  |  |

TABLE  44/Q.763

Message Type:  User part test

Message Type:  User part available

| Parameter | Reference (subclause) | Type | Length (octets) |
| --- | --- | --- | --- |
| Message type | 2.13 | F | 1 |
| Parameter compatibility information | 3.41 | O | 4-? |
| End of optional parameter | 3.20 | O | 1 |

TABLE  45/Q.763

Message Type:  Facility @

| Parameter | Reference (subclause) | Type | Length (octets) |
| --- | --- | --- | --- |
| Message type | 2.13 | F | 1 |
| Message compatibility information | 3.33 | O | 4 |
| Parameter compatibility information | 3.41 | O | 4-? |
| Remote operations | 3.48 | O | ? |
| Service activation | 3.49 | O | 3-? |
| End of optional parameter | 3.20 | O | 1 |

TABLE  46/Q.763

Message Type:  Network resource management

| Parameter | Reference (subclause) | Type | Length (octets) |
| --- | --- | --- | --- |
| Message type | 2.13 | F | 1 |
| Message compatibility information | 3.33 | O | 4 |
| Parameter compatibility information | 3.41 | O | 4-? |
| Echo control information | 3.19 | O | 3 |
| Transmission medium used | 3.55 | O | 3 |
| End of optional parameter | 3.20 | O | 1 |

TABLE  47/Q.763

Message Type:  Identification request

TABLE  48/Q.763

| Parameter name | Reference (subclause) | Type | Length (octets) |
| --- | --- | --- | --- |
| Message type | 2.13 | F | 1 |
| MCID request indicators | 3.31 | O | 3-? |
| Message compatibility information | 3.33 | O | 4-? |
| Parameter compatibility information | 3.41 | O | 4-? |
| End of optional parameter | 3.20 | O | 1 |

Message Type:  Identification response

| Parameter name | Reference (subclause) | Type | Length (octets) |
| --- | --- | --- | --- |
| Message type | 2.13 | F | 1 |
| MCID response indicator | 3.32 | O | 3-? |
| Message compatibility information | 3.33 | O | 4-? |
| Parameter compatibility information | 3.41 | O | 4-? |
| Calling party number | 3.10 | O | 5-12 |
| Access transport | 3.33 | O | 3-? |
| Generic number (Note) | 3.26 | O | 5-13 |
| End of optional parameter | 3.20 | O | 1 |
| NOTE – This parameter may be repeated. |  |  |  |

TABLE  49/Q.763

Message Type:  Segmentation

| Parameter | Reference (subclause) | Type | Length (octets) |
| --- | --- | --- | --- |
| Message type | 2.13 | F | 1 |
| Access transport | 3.33 | O | 3-? |
| User-to-user information | 3.61 | O | 3-131 |
| Message compatibility information | 3.33 | O | 4-5 |
| Generic digit (Note) | 3.24 | O | ? |
| Generic notification (Note) | 3.25 | O | 3 |
| Generic number (Note) | 3.26 | O | 5-13 |
| End of optional parameter | 3.20 | O | 1 |
| NOTE – This parameter may be repeated. |  |  |  |

1) 	The symbol @ in this Recommendation means for national use only.