ifEntry OBJECT-TYPE

SYNTAX IfEntry

ACCESS not-accessible

STATUS mandatory

DESCRIPTION

"An interface entry containing objects at the subnetwork layer and below for a particular interface."

INDEX { ifIndex }

::= { ifTable 1 }

IfEntry ::=

SEQUENCE {

ifIndex

INTEGER,

ifDescr

DisplayString,

ifType

INTEGER,

ifMtu

INTEGER,

ifSpeed

Gauge,

ifPhysAddress

PhysAddress,

ifAdminStatus

INTEGER,

ifOperStatus

INTEGER,

ifLastChange

TimeTicks,

ifInOctets

Counter,

ifInUcastPkts

Counter,

ifInNUcastPkts

Counter,

ifInDiscards

Counter,

ifInErrors

Counter,

ifInUnknownProtos

Counter,

ifOutOctets

Counter,

ifOutUcastPkts

Counter,

ifOutNUcastPkts

Counter,

ifOutDiscards

Counter,

ifOutErrors

Counter,

ifOutQLen

Gauge,

ifSpecific

OBJECT IDENTIFIER

}

|  | OBJECT-TYPE | SYNTAX | ACCESS | STATUS | DESCRIPTION | ::= {ifEntry } |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | ifIndex | INTEGER | read-only | mandatory | "A unique value for each interface. Its value ranges between 1 and the value of ifNumber. The value for each interface must remain constant at least from one re-initialization of the entity's network management system to the next re-initialization." | 1 |
| 2 | ifDescr | DisplayString (SIZE (0..255)) |  |  | "A textual string containing information about the interface. This string should include the name of the manufacturer, the product name and the version of the hardware interface." | 2 |
| 3 | ifType | INTEGER other(1), -- none of the following regular1822(2), hdh1822(3), ddn-x25(4), rfc877-x25(5), ethernet-csmacd(6), iso88023-csmacd(7), iso88024-tokenBus(8), iso88025-tokenRing(9), iso88026-man(10), starLan(11), proteon-10Mbit(12), proteon-80Mbit(13), hyperchannel(14), fddi(15), lapb(16), sdlc(17), ds1(18), -- T-1 e1(19), -- european equiv. of T-1 basicISDN(20), primaryISDN(21), propPointToPointSerial(22), ppp(23), softwareLoopback(24), eon(25), -- CLNP over IP [11] ethernet-3Mbit(26), nsip(27), -- XNS over IP slip(28), -- generic SLIP ultra(29), -- ULTRA technologies ds3(30), -- T-3 sip(31), -- SMDS frame-relay(32) | read-only | mandatory | "The type of interface, distinguished according to the physical/link protocol(s) immediately `below' the network layer in the protocol stack." | 3 |
| 4 | ifMtu | INTEGER | read-only | mandatory | "The size of the largest datagram which can be sent/received on the interface, specified in octets. For interfaces that are used for transmitting network datagrams, this is the size of the largest network datagram that can be sent on the interface." | 4 |
| 5 | ifSpeed | Gauge | read-only | mandatory | "An estimate of the interface's current bandwidth in bits per second. For interfaces which do not vary in bandwidth or for those where no accurate estimation can be made, this object should contain the nominal bandwidth." | 5 |
| 6 | ifPhysAddress | PhysAddress | read-only | mandatory | "The interface's address at the protocol layer immediately `below' the network layer in the protocol stack. For interfaces which do not have such an address (e.g., a serial line), this object should contain an octet string of zero length." | 6 |
| 7 | ifAdminStatus | INTEGER up(1), -- ready to pass packets down(2), testing(3) -- in some test mode | read-only | mandatory | "The desired state of the interface. The testing(3) state indicates that no operational packets can be passed." | 7 |
| 8 | ifOperStatus | INTEGER up(1), -- ready to pass packets down(2), testing(3) -- in some test mode | read-only | mandatory | "The current operational state of the interface. The testing(3) state indicates that no operational packets can be passed." | 8 |
| 9 | ifLastChange | TimeTicks | read-only | mandatory | "The value of sysUpTime at the time the interface entered its current operational state. If the current state was entered prior to the last re- initialization of the local network management subsystem, then this object contains a zero value." | 9 |
| 10 | ifInOctets | Counter | read-only | mandatory | "The total number of octets received on the interface, including framing characters." | 10 |
| 11 | ifInUcastPkts | Counter | read-only | mandatory | "The number of subnetwork-unicast packets delivered to a higher-layer protocol." | 11 |
| 12 | ifInNUcastPkts | Counter | read-only | mandatory | "The number of non-unicast (i.e., subnetwork-broadcast or subnetwork-multicast) packets delivered to a higher-layer protocol." | 12 |
| 13 | ifInDiscards | Counter | read-only | mandatory | "The number of inbound packets which were chosen to be discarded even though no errors had been detected to prevent their being deliverable to a higher-layer protocol. One possible reason for discarding such a packet could be to free up buffer space." | 13 |
| 14 | ifInErrors | Counter | read-only | mandatory | "The number of inbound packets that contained errors preventing them from being deliverable to a higher-layer protocol." | 14 |
| 15 | ifInUnknownProtos | Counter | read-only | mandatory | "The number of packets received via the interface which were discarded because of an unknown or unsupported protocol." | 15 |
| 16 | ifOutOctets | Counter | read-only | mandatory | "The total number of octets transmitted out of the interface, including framing characters." | 16 |
| 17 | ifOutUcastPkts | Counter | read-only | mandatory | "The total number of packets that higher-level protocols requested be transmitted to a subnetwork-unicast address, including those that were discarded or not sent." | 17 |
| 18 | ifOutNUcastPkts | Counter | read-only | mandatory | "The total number of packets that higher-level protocols requested be transmitted to a non-unicast (i.e., a subnetwork-broadcast or subnetwork-multicast) address, including those that were discarded or not sent." | 18 |
| 19 | ifOutDiscards | Counter | read-only | mandatory | "The number of outbound packets which were chosen to be discarded even though no errors had been detected to prevent their being transmitted. One possible reason for discarding such a packet could be to free up buffer space." | 19 |
| 20 | ifOutErrors | Counter | read-only | mandatory | "The number of outbound packets that could not be transmitted because of errors." | 20 |
| 21 | ifOutQLen | Gauge | read-only | mandatory | "The length of the output packet queue (in packets)." | 21 |
| 22 | ifSpecific | OBJECT IDENTIFIER | read-only | mandatory | "A reference to MIB definitions specific to the particular media being used to realize the interface. For example, if the interface is realized by an ethernet, then the value of this object refers to a document defining objects specific to ethernet. If this information is not present, its value should be set to the OBJECT IDENTIFIER { 0 0 }, which is a syntatically valid object identifier, and any conformant implementation of ASN.1 and BER must be able to generate and recognize this value." | 22 |