-- groups in MIB-II

system       OBJECT IDENTIFIER ::= { mib-2 1 }

interfaces   OBJECT IDENTIFIER ::= { mib-2 2 }

at           OBJECT IDENTIFIER ::= { mib-2 3 }

ip           OBJECT IDENTIFIER ::= { mib-2 4 }

icmp         OBJECT IDENTIFIER ::= { mib-2 5 }

tcp          OBJECT IDENTIFIER ::= { mib-2 6 }

udp          OBJECT IDENTIFIER ::= { mib-2 7 }

egp          OBJECT IDENTIFIER ::= { mib-2 8 }

-- historical (some say hysterical)

-- cmot      OBJECT IDENTIFIER ::= { mib-2 9 }

transmission OBJECT IDENTIFIER ::= { mib-2 10 }

snmp         OBJECT IDENTIFIER ::= { mib-2 11 }

-- the Interfaces group

-- Implementation of the Interfaces group is mandatory for all systems.

ifNumber OBJECT-TYPE

SYNTAX  INTEGER

ACCESS  read-only

STATUS  mandatory

DESCRIPTION

"The number of network interfaces (regardless of

their current state) present on this system."

::= { interfaces 1 }

-- the Interfaces table

-- The Interfaces table contains information on the entity's

-- interfaces.  Each interface is thought of as being

-- attached to a `subnetwork'.  Note that this term should

-- not be confused with `subnet' which refers to an

-- addressing partitioning scheme used in the Internet suite of protocols.

ifTable OBJECT-TYPE

SYNTAX  SEQUENCE OF IfEntry

ACCESS  not-accessible

STATUS  mandatory

DESCRIPTION

"A list of interface entries.  The number of

entries is given by the value of ifNumber."

::= { interfaces 2 }

ifEntry OBJECT-TYPE

SYNTAX  IfEntry

ACCESS  not-accessible

STATUS  mandatory

DESCRIPTION

"An interface entry containing objects at the

subnetwork layer and below for a particular interface."

INDEX   { ifIndex }

::= { ifTable 1 }

IfEntry ::=

SEQUENCE {

ifIndex

INTEGER,

ifDescr

DisplayString,

ifType

INTEGER,

ifMtu

INTEGER,

ifSpeed

Gauge,

ifPhysAddress

PhysAddress,

ifAdminStatus

INTEGER,

ifOperStatus

INTEGER,

ifLastChange

TimeTicks,

ifInOctets

Counter,

ifInUcastPkts

Counter,

ifInNUcastPkts

Counter,

ifInDiscards

Counter,

ifInErrors

Counter,

ifInUnknownProtos

Counter,

ifOutOctets

Counter,

ifOutUcastPkts

Counter,

ifOutNUcastPkts

Counter,

ifOutDiscards

Counter,

ifOutErrors

Counter,

ifOutQLen

Gauge,

ifSpecific

OBJECT IDENTIFIER

}

ifIndex OBJECT-TYPE

SYNTAX  INTEGER

ACCESS  read-only

STATUS  mandatory

DESCRIPTION

"A unique value for each interface.  Its value

ranges between 1 and the value of ifNumber.  The

value for each interface must remain constant at

least from one re-initialization of the entity's

network management system to the next re-

initialization."

::= { ifEntry 1 }

ifDescr OBJECT-TYPE

SYNTAX  DisplayString (SIZE (0..255))

ACCESS  read-only

STATUS  mandatory

DESCRIPTION

"A textual string containing information about the

interface.  This string should include the name of

the manufacturer, the product name and the version

of the hardware interface."

::= { ifEntry 2 }

ifType OBJECT-TYPE

SYNTAX  INTEGER {

other(1),          -- none of the following

regular1822(2),

hdh1822(3),

ddn-x25(4),

rfc877-x25(5),

ethernet-csmacd(6),

iso88023-csmacd(7),

iso88024-tokenBus(8),

iso88025-tokenRing(9),

iso88026-man(10),

starLan(11),

proteon-10Mbit(12),

proteon-80Mbit(13),

hyperchannel(14),

fddi(15),

lapb(16),

sdlc(17),

ds1(18),           -- T-1

e1(19),            -- european equiv. of T-1

basicISDN(20),

primaryISDN(21),   -- proprietary serial

propPointToPointSerial(22),

ppp(23),

softwareLoopback(24),

eon(25),            -- CLNP over IP [11]

ethernet-3Mbit(26),

nsip(27),           -- XNS over IP

slip(28),           -- generic SLIP

ultra(29),          -- ULTRA technologies

ds3(30),            -- T-3

sip(31),            -- SMDS

frame-relay(32)

}

ACCESS  read-only

STATUS  mandatory

DESCRIPTION

"The type of interface, distinguished according to

the physical/link protocol(s) immediately `below'

the network layer in the protocol stack."

::= { ifEntry 3 }

ifMtu OBJECT-TYPE

SYNTAX  INTEGER

ACCESS  read-only

STATUS  mandatory

DESCRIPTION

"The size of the largest datagram which can be

sent/received on the interface, specified in

octets.  For interfaces that are used for

transmitting network datagrams, this is the size

of the largest network datagram that can be sent

on the interface."

::= { ifEntry 4 }

ifSpeed OBJECT-TYPE

SYNTAX  Gauge

ACCESS  read-only

STATUS  mandatory

DESCRIPTION

"An estimate of the interface's current bandwidth

in bits per second.  For interfaces which do not

vary in bandwidth or for those where no accurate

estimation can be made, this object should contain

the nominal bandwidth."

::= { ifEntry 5 }

ifPhysAddress OBJECT-TYPE

SYNTAX  PhysAddress

ACCESS  read-only

STATUS  mandatory

DESCRIPTION

"The interface's address at the protocol layer

immediately `below' the network layer in the

protocol stack.  For interfaces which do not have

such an address (e.g., a serial line), this object

should contain an octet string of zero length."

::= { ifEntry 6 }

ifAdminStatus OBJECT-TYPE

SYNTAX  INTEGER {

up(1),       -- ready to pass packets

down(2),

testing(3)   -- in some test mode

}

ACCESS  read-write

STATUS  mandatory

DESCRIPTION

"The desired state of the interface.  The

testing(3) state indicates that no operational

packets can be passed."

::= { ifEntry 7 }

ifOperStatus OBJECT-TYPE

SYNTAX  INTEGER {

up(1),       -- ready to pass packets

down(2),

testing(3)   -- in some test mode

}

ACCESS  read-only

STATUS  mandatory

DESCRIPTION

"The current operational state of the interface.

The testing(3) state indicates that no operational

packets can be passed."

::= { ifEntry 8 }

ifLastChange OBJECT-TYPE

SYNTAX  TimeTicks

ACCESS  read-only

STATUS  mandatory

DESCRIPTION

"The value of sysUpTime at the time the interface

entered its current operational state.  If the

current state was entered prior to the last re-

initialization of the local network management

subsystem, then this object contains a zero value."

::= { ifEntry 9 }

ifInOctets OBJECT-TYPE

SYNTAX  Counter

ACCESS  read-only

STATUS  mandatory

DESCRIPTION

"The total number of octets received on the

interface, including framing characters."

::= { ifEntry 10 }

ifInUcastPkts OBJECT-TYPE

SYNTAX  Counter

ACCESS  read-only

STATUS  mandatory

DESCRIPTION

"The number of subnetwork-unicast packets

delivered to a higher-layer protocol."

::= { ifEntry 11 }

ifInNUcastPkts OBJECT-TYPE

SYNTAX  Counter

ACCESS  read-only

STATUS  mandatory

DESCRIPTION

"The number of non-unicast (i.e., subnetwork-

broadcast or subnetwork-multicast) packets

delivered to a higher-layer protocol."

::= { ifEntry 12 }

ifInDiscards OBJECT-TYPE

SYNTAX  Counter

ACCESS  read-only

STATUS  mandatory

DESCRIPTION

"The number of inbound packets which were chosen

to be discarded even though no errors had been

detected to prevent their being deliverable to a

higher-layer protocol.  One possible reason for

discarding such a packet could be to free up

buffer space."

::= { ifEntry 13 }

ifInErrors OBJECT-TYPE

SYNTAX  Counter

ACCESS  read-only

STATUS  mandatory

DESCRIPTION

"The number of inbound packets that contained

errors preventing them from being deliverable to a

higher-layer protocol."

::= { ifEntry 14 }

ifInUnknownProtos OBJECT-TYPE

SYNTAX  Counter

ACCESS  read-only

STATUS  mandatory

DESCRIPTION

"The number of packets received via the interface

which were discarded because of an unknown or

unsupported protocol."

::= { ifEntry 15 }

ifOutOctets OBJECT-TYPE

SYNTAX  Counter

ACCESS  read-only

STATUS  mandatory

DESCRIPTION

"The total number of octets transmitted out of the

interface, including framing characters."

::= { ifEntry 16 }

ifOutUcastPkts OBJECT-TYPE

SYNTAX  Counter

ACCESS  read-only

STATUS  mandatory

DESCRIPTION

"The total number of packets that higher-level

protocols requested be transmitted to a

subnetwork-unicast address, including those that

were discarded or not sent."

::= { ifEntry 17 }

ifOutNUcastPkts OBJECT-TYPE

SYNTAX  Counter

ACCESS  read-only

STATUS  mandatory

DESCRIPTION

"The total number of packets that higher-level

protocols requested be transmitted to a non-

unicast (i.e., a subnetwork-broadcast or

subnetwork-multicast) address, including those

that were discarded or not sent."

::= { ifEntry 18 }

ifOutDiscards OBJECT-TYPE

SYNTAX  Counter

ACCESS  read-only

STATUS  mandatory

DESCRIPTION

"The number of outbound packets which were chosen

to be discarded even though no errors had been

detected to prevent their being transmitted.  One

possible reason for discarding such a packet could

be to free up buffer space."

::= { ifEntry 19 }

ifOutErrors OBJECT-TYPE

SYNTAX  Counter

ACCESS  read-only

STATUS  mandatory

DESCRIPTION

"The number of outbound packets that could not be

transmitted because of errors."

::= { ifEntry 20 }

ifOutQLen OBJECT-TYPE

SYNTAX  Gauge

ACCESS  read-only

STATUS  mandatory

DESCRIPTION

"The length of the output packet queue (in

packets)."

::= { ifEntry 21 }

ifSpecific OBJECT-TYPE

SYNTAX  OBJECT IDENTIFIER

ACCESS  read-only

STATUS  mandatory

DESCRIPTION

"A reference to MIB definitions specific to the

particular media being used to realize the

interface.  For example, if the interface is

realized by an ethernet, then the value of this

object refers to a document defining objects

specific to ethernet.  If this information is not

present, its value should be set to the OBJECT

IDENTIFIER { 0 0 }, which is a syntatically valid

object identifier, and any conformant

implementation of ASN.1 and BER must be able to

generate and recognize this value."

::= { ifEntry 22 }