RFC1213

Рис. 1 – Идентификационное дерево объектов ASN.1

Ниже приводятся спецификации для поддерева MIB-2 (1.3.6.1.2.1)

-MIB DEFINITIONS ::= BEGIN

IMPORTS

mgmt, NetworkAddress, IpAddress, Counter, Gauge, TimeTicks

FROM RFC1155-SMI

OBJECT-TYPE

FROM RFC-1212;

--  This MIB module uses the extended OBJECT-TYPE macro as defined in [14];

--  MIB-II (same prefix as MIB-I)

mib-2      OBJECT IDENTIFIER ::= { mgmt 1 }	 - 1.3.6.1.2.1

-- textual conventions

DisplayString ::=

OCTET STRING

-- This data type is used to model textual information taken

-- from the NVT ASCII character set.  By convention, objects

-- with this syntax are declared as having

--

--      SIZE (0..255)

PhysAddress ::=

OCTET STRING

-- This data type is used to model media addresses.  For many

-- types of media, this will be in a binary representation.

-- For example, an ethernet address would be represented as

-- a string of 6 octets.

-- groups in MIB-II – 11 объектов в этой группе - 1.3.6.1.2.1.1…11

system       OBJECT IDENTIFIER ::= { mib-2 1 }

interfaces   OBJECT IDENTIFIER ::= { mib-2 2 }

at           OBJECT IDENTIFIER ::= { mib-2 3 }

ip           OBJECT IDENTIFIER ::= { mib-2 4 }

icmp         OBJECT IDENTIFIER ::= { mib-2 5 }

tcp          OBJECT IDENTIFIER ::= { mib-2 6 }

udp          OBJECT IDENTIFIER ::= { mib-2 7 }

egp          OBJECT IDENTIFIER ::= { mib-2 8 }

-- historical (some say hysterical)

-- cmot      OBJECT IDENTIFIER ::= { mib-2 9 }

transmission OBJECT IDENTIFIER ::= { mib-2 10 }

snmp         OBJECT IDENTIFIER ::= { mib-2 11 }

-- the System group - 7 объектов в этой группе -1.3.6.1.2.1.1.1…7

-- Implementation of the System group is mandatory for all

-- systems.  If an agent is not configured to have a value

-- for any of these variables, a string of length 0 is returned.

sysDescr OBJECT-TYPE

SYNTAX  DisplayString (SIZE (0..255))

ACCESS  read-only

STATUS  mandatory

DESCRIPTION

"A textual description of the entity.  This value

should include the full name and version

identification of the system's hardware type,

software operating-system, and networking

software.  It is mandatory that this only contain

printable ASCII characters."

::= { system 1 }

sysObjectID OBJECT-TYPE

SYNTAX  OBJECT IDENTIFIER

ACCESS  read-only

STATUS  mandatory

DESCRIPTION

"The vendor's authoritative identification of the

network management subsystem contained in the

entity.  This value is allocated within the SMI

enterprises subtree (1.3.6.1.4.1) and provides an

easy and unambiguous means for determining `what

kind of box' is being managed.  For example, if

vendor `Flintstones, Inc.' was assigned the

subtree 1.3.6.1.4.1.4242, it could assign the

identifier 1.3.6.1.4.1.4242.1.1 to its `Fred Router'."

::= { system 2 }

sysUpTime OBJECT-TYPE

SYNTAX  TimeTicks

ACCESS  read-only

STATUS  mandatory

DESCRIPTION

"The time (in hundredths of a second) since the

network management portion of the system was last

re-initialized."

::= { system 3 }

sysContact OBJECT-TYPE

SYNTAX  DisplayString (SIZE (0..255))

ACCESS  read-write

STATUS  mandatory

DESCRIPTION

"The textual identification of the contact person

for this managed node, together with information

on how to contact this person."

::= { system 4 }

sysName OBJECT-TYPE

SYNTAX  DisplayString (SIZE (0..255))

ACCESS  read-write

STATUS  mandatory

DESCRIPTION

"An administratively-assigned name for this

managed node.  By convention, this is the node's

fully-qualified domain name."

::= { system 5 }

sysLocation OBJECT-TYPE

SYNTAX  DisplayString (SIZE (0..255))

ACCESS  read-write

STATUS  mandatory

DESCRIPTION

"The physical location of this node (e.g.,

`telephone closet, 3rd floor')."

::= { system 6 }

sysServices OBJECT-TYPE

SYNTAX  INTEGER (0..127)

ACCESS  read-only

STATUS  mandatory

DESCRIPTION

"A value which indicates the set of services that

this entity primarily offers.

The value is a sum.  This sum initially takes the

value zero, Then, for each layer, L, in the range

1 through 7, that this node performs transactions

for, 2 raised to (L - 1) is added to the sum.  For

example, a node which performs primarily routing

functions would have a value of 4 (2^(3-1)).  In

contrast, a node which is a host offering

application services would have a value of 72

(2^(4-1) + 2^(7-1)).  Note that in the context of

the Internet suite of protocols, values should be

calculated accordingly:

layer  functionality

1  physical (e.g., repeaters)

2  datalink/subnetwork (e.g., bridges)

3  internet (e.g., IP gateways)

4  end-to-end  (e.g., IP hosts)

7  applications (e.g., mail relays)

For systems including OSI protocols, layers 5 and

6 may also be counted."

::= { system 7 }

-- the Interfaces group - 2 объекта в этой группе -1.3.6.1.2.1.1.1…2

-- Implementation of the Interfaces group is mandatory for all systems.

ifNumber OBJECT-TYPE

SYNTAX  INTEGER

ACCESS  read-only

STATUS  mandatory

DESCRIPTION

"The number of network interfaces (regardless of

their current state) present on this system."

::= { interfaces 1 }

-- the Interfaces table

-- The Interfaces table contains information on the entity's

-- interfaces.  Each interface is thought of as being

-- attached to a `subnetwork'.  Note that this term should

-- not be confused with `subnet' which refers to an

-- addressing partitioning scheme used in the Internet suite of protocols.

ifTable OBJECT-TYPE - 1 объект в этой группе -1.3.6.1.2.1.1.1.1

SYNTAX  SEQUENCE OF IfEntry

ACCESS  not-accessible

STATUS  mandatory

DESCRIPTION

"A list of interface entries.  The number of

entries is given by the value of ifNumber."

::= { interfaces 2 }

ifEntry OBJECT-TYPE - 22 объекта в этой группе -1.3.6.1.2.1.1.1.1.1…22

SYNTAX  IfEntry

ACCESS  not-accessible

STATUS  mandatory

DESCRIPTION

"An interface entry containing objects at the

subnetwork layer and below for a particular interface."

INDEX   { ifIndex }

::= { ifTable 1 }

IfEntry ::=

SEQUENCE {

ifIndex

INTEGER,

ifDescr

DisplayString,

ifType

INTEGER,

ifMtu

INTEGER,

ifSpeed

Gauge,

ifPhysAddress

PhysAddress,

ifAdminStatus

INTEGER,

ifOperStatus

INTEGER,

ifLastChange

TimeTicks,

ifInOctets

Counter,

ifInUcastPkts

Counter,

ifInNUcastPkts

Counter,

ifInDiscards

Counter,

ifInErrors

Counter,

ifInUnknownProtos

Counter,

ifOutOctets

Counter,

ifOutUcastPkts

Counter,

ifOutNUcastPkts

Counter,

ifOutDiscards

Counter,

ifOutErrors

Counter,

ifOutQLen

Gauge,

ifSpecific

OBJECT IDENTIFIER

}

ifIndex OBJECT-TYPE

SYNTAX  INTEGER

ACCESS  read-only

STATUS  mandatory

DESCRIPTION

"A unique value for each interface.  Its value

ranges between 1 and the value of ifNumber.  The

value for each interface must remain constant at

least from one re-initialization of the entity's

network management system to the next reinitialization."

::= { ifEntry 1 }

ifDescr OBJECT-TYPE

SYNTAX  DisplayString (SIZE (0..255))

ACCESS  read-only

STATUS  mandatory

DESCRIPTION

"A textual string containing information about the

interface.  This string should include the name of

the manufacturer, the product name and the version

of the hardware interface."

::= { ifEntry 2 }

ifType OBJECT-TYPE

SYNTAX  INTEGER {

other(1),          -- none of the following

regular1822(2),

hdh1822(3),

ddn-x25(4),

rfc877-x25(5),

ethernet-csmacd(6),

iso88023-csmacd(7),

iso88024-tokenBus(8),

iso88025-tokenRing(9),

iso88026-man(10),

starLan(11),

proteon-10Mbit(12),

proteon-80Mbit(13),

hyperchannel(14),

fddi(15),

lapb(16),

sdlc(17),

ds1(18),           -- T-1

e1(19),            -- european equiv. of T-1

basicISDN(20),

primaryISDN(21),   -- proprietary serial

propPointToPointSerial(22),

ppp(23),

softwareLoopback(24),

eon(25),            -- CLNP over IP [11]

ethernet-3Mbit(26),

nsip(27),           -- XNS over IP

slip(28),           -- generic SLIP

ultra(29),          -- ULTRA technologies

ds3(30),            -- T-3

sip(31),            -- SMDS

frame-relay(32)

}

ACCESS  read-only

STATUS  mandatory

DESCRIPTION

"The type of interface, distinguished according to

the physical/link protocol(s) immediately `below'

the network layer in the protocol stack."

::= { ifEntry 3 }

ifMtu OBJECT-TYPE

SYNTAX  INTEGER

ACCESS  read-only

STATUS  mandatory

DESCRIPTION

"The size of the largest datagram which can be

sent/received on the interface, specified in

octets.  For interfaces that are used for

transmitting network datagrams, this is the size

of the largest network datagram that can be sent

on the interface."

::= { ifEntry 4 }

ifSpeed OBJECT-TYPE

SYNTAX  Gauge

ACCESS  read-only

STATUS  mandatory

DESCRIPTION

"An estimate of the interface's current bandwidth

in bits per second.  For interfaces which do not

vary in bandwidth or for those where no accurate

estimation can be made, this object should contain

the nominal bandwidth."

::= { ifEntry 5 }

ifPhysAddress OBJECT-TYPE

SYNTAX  PhysAddress

ACCESS  read-only

STATUS  mandatory

DESCRIPTION

"The interface's address at the protocol layer

immediately `below' the network layer in the

protocol stack.  For interfaces which do not have

such an address (e.g., a serial line), this object

should contain an octet string of zero length."

::= { ifEntry 6 }

ifAdminStatus OBJECT-TYPE

SYNTAX  INTEGER {

up(1),       -- ready to pass packets

down(2),

testing(3)   -- in some test mode

}

ACCESS  read-write

STATUS  mandatory

DESCRIPTION

"The desired state of the interface.  The

testing(3) state indicates that no operational

packets can be passed."

::= { ifEntry 7 }

ifOperStatus OBJECT-TYPE

SYNTAX  INTEGER {

up(1),       -- ready to pass packets

down(2),

testing(3)   -- in some test mode

}

ACCESS  read-only

STATUS  mandatory

DESCRIPTION

"The current operational state of the interface.

The testing(3) state indicates that no operational

packets can be passed."

::= { ifEntry 8 }

ifLastChange OBJECT-TYPE

SYNTAX  TimeTicks

ACCESS  read-only

STATUS  mandatory

DESCRIPTION

"The value of sysUpTime at the time the interface

entered its current operational state.  If the

current state was entered prior to the last re-

initialization of the local network management

subsystem, then this object contains a zero

value."

::= { ifEntry 9 }

ifInOctets OBJECT-TYPE

SYNTAX  Counter

ACCESS  read-only

STATUS  mandatory

DESCRIPTION

"The total number of octets received on the

interface, including framing characters."

::= { ifEntry 10 }

ifInUcastPkts OBJECT-TYPE

SYNTAX  Counter

ACCESS  read-only

STATUS  mandatory

DESCRIPTION

"The number of subnetwork-unicast packets

delivered to a higher-layer protocol."

::= { ifEntry 11 }

ifInNUcastPkts OBJECT-TYPE

SYNTAX  Counter

ACCESS  read-only

STATUS  mandatory

DESCRIPTION

"The number of non-unicast (i.e., subnetwork-

broadcast or subnetwork-multicast) packets

delivered to a higher-layer protocol."

::= { ifEntry 12 }

ifInDiscards OBJECT-TYPE

SYNTAX  Counter

ACCESS  read-only

STATUS  mandatory

DESCRIPTION

"The number of inbound packets which were chosen

to be discarded even though no errors had been

detected to prevent their being deliverable to a

higher-layer protocol.  One possible reason for

discarding such a packet could be to free up

buffer space."

::= { ifEntry 13 }

ifInErrors OBJECT-TYPE

SYNTAX  Counter

ACCESS  read-only

STATUS  mandatory

DESCRIPTION

"The number of inbound packets that contained

errors preventing them from being deliverable to a

higher-layer protocol."

::= { ifEntry 14 }

ifInUnknownProtos OBJECT-TYPE

SYNTAX  Counter

ACCESS  read-only

STATUS  mandatory

DESCRIPTION

"The number of packets received via the interface

which were discarded because of an unknown or

unsupported protocol."

::= { ifEntry 15 }

ifOutOctets OBJECT-TYPE

SYNTAX  Counter

ACCESS  read-only

STATUS  mandatory

DESCRIPTION

"The total number of octets transmitted out of the

interface, including framing characters."

::= { ifEntry 16 }

ifOutUcastPkts OBJECT-TYPE

SYNTAX  Counter

ACCESS  read-only

STATUS  mandatory

DESCRIPTION

"The total number of packets that higher-level

protocols requested be transmitted to a

subnetwork-unicast address, including those that

were discarded or not sent."

::= { ifEntry 17 }

ifOutNUcastPkts OBJECT-TYPE

SYNTAX  Counter

ACCESS  read-only

STATUS  mandatory

DESCRIPTION

"The total number of packets that higher-level

protocols requested be transmitted to a non-

unicast (i.e., a subnetwork-broadcast or

subnetwork-multicast) address, including those

that were discarded or not sent."

::= { ifEntry 18 }

ifOutDiscards OBJECT-TYPE

SYNTAX  Counter

ACCESS  read-only

STATUS  mandatory

DESCRIPTION

"The number of outbound packets which were chosen

to be discarded even though no errors had been

detected to prevent their being transmitted.  One

possible reason for discarding such a packet could

be to free up buffer space."

::= { ifEntry 19 }

ifOutErrors OBJECT-TYPE

SYNTAX  Counter

ACCESS  read-only

STATUS  mandatory

DESCRIPTION

"The number of outbound packets that could not be

transmitted because of errors."

::= { ifEntry 20 }

ifOutQLen OBJECT-TYPE

SYNTAX  Gauge

ACCESS  read-only

STATUS  mandatory

DESCRIPTION

"The length of the output packet queue (in

packets)."

::= { ifEntry 21 }

ifSpecific OBJECT-TYPE

SYNTAX  OBJECT IDENTIFIER

ACCESS  read-only

STATUS  mandatory

DESCRIPTION

"A reference to MIB definitions specific to the

particular media being used to realize the

interface.  For example, if the interface is

realized by an ethernet, then the value of this

object refers to a document defining objects

specific to ethernet.  If this information is not

present, its value should be set to the OBJECT

IDENTIFIER { 0 0 }, which is a syntatically valid

object identifier, and any conformant

implementation of ASN.1 and BER must be able to

generate and recognize this value."

::= { ifEntry 22 }

-- the Address Translation group - 1 объект в этой группе -1.3.6.1.2.1.1.1

-- Implementation of the Address Translation group is

-- mandatory for all systems.  Note however that this group

-- is deprecated by MIB-II. That is, it is being included

-- solely for compatibility with MIB-I nodes, and will most

-- likely be excluded from MIB-III nodes.  From MIB-II and

-- onwards, each network protocol group contains its own

-- address translation tables.

-- The Address Translation group contains one table which is

-- the union across all interfaces of the translation tables

-- for converting a NetworkAddress (e.g., an IP address) into

-- a subnetwork-specific address.  For lack of a better term,

-- this document refers to such a subnetwork-specific address

-- as a `physical' address.

-- Examples of such translation tables are: for broadcast

-- media where ARP is in use, the translation table is

-- equivalent to the ARP cache; or, on an X.25 network where

-- non-algorithmic translation to X.121 addresses is

-- required, the translation table contains the

-- NetworkAddress to X.121 address equivalences.

atTable OBJECT-TYPE - 1 объект в этой группе -1.3.6.1.2.1.1.1.1

SYNTAX  SEQUENCE OF AtEntry

ACCESS  not-accessible

STATUS  deprecated

DESCRIPTION

"The Address Translation tables contain the

NetworkAddress to `physical' address equivalences.

Some interfaces do not use translation tables for

determining address equivalences (e.g., DDN-X.25

has an algorithmic method); if all interfaces are

of this type, then the Address Translation table

is empty, i.e., has zero entries."

::= { at 1 }

atEntry OBJECT-TYPE - 3 объекта в этой группе -1.3.6.1.2.1.1.1.1.1…3

SYNTAX  AtEntry

ACCESS  not-accessible

STATUS  deprecated

DESCRIPTION

"Each entry contains one NetworkAddress to

`physical' address equivalence."

INDEX   { atIfIndex,

atNetAddress }

::= { atTable 1 }

atEntry ::=

SEQUENCE {

atIfIndex

INTEGER,

atPhysAddress

PhysAddress,

atNetAddress

NetworkAddress

}

atIfIndex OBJECT-TYPE

SYNTAX  INTEGER

ACCESS  read-write

STATUS  deprecated

DESCRIPTION

"The interface on which this entry's equivalence

is effective.  The interface identified by a

particular value of this index is the same

interface as identified by the same value of

ifIndex."

::= { atEntry 1 }

atPhysAddress OBJECT-TYPE

SYNTAX  PhysAddress

ACCESS  read-write

STATUS  deprecated

DESCRIPTION

"The media-dependent `physical' address.

Setting this object to a null string (one of zero

length) has the effect of invaliding the

corresponding entry in the atTable object.  That

is, it effectively dissasociates the interface

identified with said entry from the mapping

identified with said entry.  It is an

implementation-specific matter as to whether the

agent removes an invalidated entry from the table.

Accordingly, management stations must be prepared

to receive tabular information from agents that

corresponds to entries not currently in use.

Proper interpretation of such entries requires

examination of the relevant atPhysAddress object."

::= { atEntry 2 }

atNetAddress OBJECT-TYPE

SYNTAX  NetworkAddress

ACCESS  read-write

STATUS  deprecated

DESCRIPTION

"The NetworkAddress (e.g., the IP address)

corresponding to the media-dependent `physical'

address."

::= { atEntry 3 }

-- the IP group - - 23 объекта в этой группе -1.3.6.1.2.1.1.1…23

-- Implementation of the IP group is mandatory for all

-- systems.

ipForwarding OBJECT-TYPE

SYNTAX  INTEGER {

forwarding(1),    -- acting as a gateway

not-forwarding(2) -- NOT acting as a gateway

}

ACCESS  read-write

STATUS  mandatory

DESCRIPTION

"The indication of whether this entity is acting

as an IP gateway in respect to the forwarding of

datagrams received by, but not addressed to, this

entity.  IP gateways forward datagrams.  IP hosts

do not (except those source-routed via the host).

Note that for some managed nodes, this object may

take on only a subset of the values possible.

Accordingly, it is appropriate for an agent to

return a `badValue' response if a management

station attempts to change this object to an

inappropriate value."

::= { ip 1 }

ipDefaultTTL OBJECT-TYPE

SYNTAX  INTEGER

ACCESS  read-write

STATUS  mandatory

DESCRIPTION

"The default value inserted into the Time-To-Live

field of the IP header of datagrams originated at

this entity, whenever a TTL value is not supplied

by the transport layer protocol."

::= { ip 2 }

ipInReceives OBJECT-TYPE

SYNTAX  Counter

ACCESS  read-only

STATUS  mandatory

DESCRIPTION

"The total number of input datagrams received from

interfaces, including those received in error."

::= { ip 3 }

ipInHdrErrors OBJECT-TYPE

SYNTAX  Counter

ACCESS  read-only

STATUS  mandatory

DESCRIPTION

"The number of input datagrams discarded due to

errors in their IP headers, including bad

checksums, version number mismatch, other format

errors, time-to-live exceeded, errors discovered

in processing their IP options, etc."

::= { ip 4 }

ipInAddrErrors OBJECT-TYPE

SYNTAX  Counter

ACCESS  read-only

STATUS  mandatory

DESCRIPTION

"The number of input datagrams discarded because

the IP address in their IP header's destination

field was not a valid address to be received at

this entity.  This count includes invalid

addresses (e.g., 0.0.0.0) and addresses of

unsupported Classes (e.g., Class E).  For entities

which are not IP Gateways and therefore do not

forward datagrams, this counter includes datagrams

discarded because the destination address was not

a local address."

::= { ip 5 }

ipForwDatagrams OBJECT-TYPE

SYNTAX  Counter

ACCESS  read-only

STATUS  mandatory

DESCRIPTION

"The number of input datagrams for which this

entity was not their final IP destination, as a

result of which an attempt was made to find a

route to forward them to that final destination.

In entities which do not act as IP Gateways, this

counter will include only those packets which were

Source-Routed via this entity, and the Source-

Route option processing was successful."

::= { ip 6 }

ipInUnknownProtos OBJECT-TYPE

SYNTAX  Counter

ACCESS  read-only

STATUS  mandatory

DESCRIPTION

"The number of locally-addressed datagrams

received successfully but discarded because of an

unknown or unsupported protocol."

::= { ip 7 }

ipInDiscards OBJECT-TYPE

SYNTAX  Counter

ACCESS  read-only

STATUS  mandatory

DESCRIPTION

"The number of input IP datagrams for which no

problems were encountered to prevent their

continued processing, but which were discarded

(e.g., for lack of buffer space).  Note that this

counter does not include any datagrams discarded

while awaiting re-assembly."

::= { ip 8 }

ipInDelivers OBJECT-TYPE

SYNTAX  Counter

ACCESS  read-only

STATUS  mandatory

DESCRIPTION

"The total number of input datagrams successfully

delivered to IP user-protocols (including ICMP)."

::= { ip 9 }

ipOutRequests OBJECT-TYPE

SYNTAX  Counter

ACCESS  read-only

STATUS  mandatory

DESCRIPTION

"The total number of IP datagrams which local IP

user-protocols (including ICMP) supplied to IP in

requests for transmission.  Note that this counter

does not include any datagrams counted in

ipForwDatagrams."

::= { ip 10 }

ipOutDiscards OBJECT-TYPE

SYNTAX  Counter

ACCESS  read-only

STATUS  mandatory

DESCRIPTION

"The number of output IP datagrams for which no

problem was encountered to prevent their

transmission to their destination, but which were

discarded (e.g., for lack of buffer space).  Note

that this counter would include datagrams counted

in ipForwDatagrams if any such packets met this

(discretionary) discard criterion."

::= { ip 11 }

ipOutNoRoutes OBJECT-TYPE

SYNTAX  Counter

ACCESS  read-only

STATUS  mandatory

DESCRIPTION

"The number of IP datagrams discarded because no

route could be found to transmit them to their

destination.  Note that this counter includes any

packets counted in ipForwDatagrams which meet this

`no-route' criterion.  Note that this includes any

datagarms which a host cannot route because all of

its default gateways are down."

::= { ip 12 }

ipReasmTimeout OBJECT-TYPE

SYNTAX  INTEGER

ACCESS  read-only

STATUS  mandatory

DESCRIPTION

"The maximum number of seconds which received

fragments are held while they are awaiting

reassembly at this entity."

::= { ip 13 }

ipReasmReqds OBJECT-TYPE

SYNTAX  Counter

ACCESS  read-only

STATUS  mandatory

DESCRIPTION

"The number of IP fragments received which needed

to be reassembled at this entity."

::= { ip 14 }

ipReasmOKs OBJECT-TYPE

SYNTAX  Counter

ACCESS  read-only

STATUS  mandatory

DESCRIPTION

"The number of IP datagrams successfully re-

assembled."

::= { ip 15 }

ipReasmFails OBJECT-TYPE

SYNTAX  Counter

ACCESS  read-only

STATUS  mandatory

DESCRIPTION

"The number of failures detected by the IP re-

assembly algorithm (for whatever reason: timed

out, errors, etc).  Note that this is not

necessarily a count of discarded IP fragments

since some algorithms (notably the algorithm in

RFC 815) can lose track of the number of fragments

by combining them as they are received."

::= { ip 16 }

ipFragOKs OBJECT-TYPE

SYNTAX  Counter

ACCESS  read-only

STATUS  mandatory

DESCRIPTION

"The number of IP datagrams that have been

successfully fragmented at this entity."

::= { ip 17 }

ipFragFails OBJECT-TYPE

SYNTAX  Counter

ACCESS  read-only

STATUS  mandatory

DESCRIPTION

"The number of IP datagrams that have been

discarded because they needed to be fragmented at

this entity but could not be, e.g., because their

Don't Fragment flag was set."

::= { ip 18 }

ipFragCreates OBJECT-TYPE

SYNTAX  Counter

ACCESS  read-only

STATUS  mandatory

DESCRIPTION

"The number of IP datagram fragments that have

been generated as a result of fragmentation at

this entity."

::= { ip 19 }

-- the IP address table

-- The IP address table contains this entity's IP addressing information.

ipAddrTable OBJECT-TYPE

SYNTAX  SEQUENCE OF IpAddrEntry

ACCESS  not-accessible

STATUS  mandatory

DESCRIPTION

"The table of addressing information relevant to

this entity's IP addresses."

::= { ip 20 }

ipAddrEntry OBJECT-TYPE

SYNTAX  IpAddrEntry

ACCESS  not-accessible

STATUS  mandatory

DESCRIPTION

"The addressing information for one of this

entity's IP addresses."

INDEX   { ipAdEntAddr }

::= { ipAddrTable 1 }

IpAddrEntry ::=

SEQUENCE {

ipAdEntAddr

IpAddress,

ipAdEntIfIndex

INTEGER,

ipAdEntNetMask

IpAddress,

ipAdEntBcastAddr

INTEGER,

ipAdEntReasmMaxSize

INTEGER (0..65535)

}

ipAdEntAddr OBJECT-TYPE

SYNTAX  IpAddress

ACCESS  read-only

STATUS  mandatory

DESCRIPTION

"The IP address to which this entry's addressing

information pertains."

::= { ipAddrEntry 1 }

ipAdEntIfIndex OBJECT-TYPE

SYNTAX  INTEGER

ACCESS  read-only

STATUS  mandatory

DESCRIPTION

"The index value which uniquely identifies the

interface to which this entry is applicable.  The

interface identified by a particular value of this

index is the same interface as identified by the

same value of ifIndex."

::= { ipAddrEntry 2 }

ipAdEntNetMask OBJECT-TYPE

SYNTAX  IpAddress

ACCESS  read-only

STATUS  mandatory

DESCRIPTION

"The subnet mask associated with the IP address of

this entry.  The value of the mask is an IP

address with all the network bits set to 1 and all

the hosts bits set to 0."

::= { ipAddrEntry 3 }

ipAdEntBcastAddr OBJECT-TYPE

SYNTAX  INTEGER

ACCESS  read-only

STATUS  mandatory

DESCRIPTION

"The value of the least-significant bit in the IP

broadcast address used for sending datagrams on

the (logical) interface associated with the IP

address of this entry.  For example, when the

Internet standard all-ones broadcast address is

used, the value will be 1.  This value applies to

both the subnet and network broadcasts addresses

used by the entity on this (logical) interface."

::= { ipAddrEntry 4 }

ipAdEntReasmMaxSize OBJECT-TYPE

SYNTAX  INTEGER (0..65535)

ACCESS  read-only

STATUS  mandatory

DESCRIPTION

"The size of the largest IP datagram which this

entity can re-assemble from incoming IP fragmented

datagrams received on this interface."

::= { ipAddrEntry 5 }

-- the IP routing table

-- The IP routing table contains an entry for each route

-- presently known to this entity.

ipRouteTable OBJECT-TYPE

SYNTAX  SEQUENCE OF IpRouteEntry

ACCESS  not-accessible

STATUS  mandatory

DESCRIPTION

"This entity's IP Routing table."

::= { ip 21 }

ipRouteEntry OBJECT-TYPE

SYNTAX  IpRouteEntry

ACCESS  not-accessible

STATUS  mandatory

DESCRIPTION

"A route to a particular destination."

INDEX   { ipRouteDest }

::= { ipRouteTable 1 }

IpRouteEntry ::=

SEQUENCE {

ipRouteDest

IpAddress,

ipRouteIfIndex

INTEGER,

ipRouteMetric1

INTEGER,

ipRouteMetric2

INTEGER,

ipRouteMetric3

INTEGER,

ipRouteMetric4

INTEGER,

ipRouteNextHop

IpAddress,

ipRouteType

INTEGER,

ipRouteProto

INTEGER,

ipRouteAge

INTEGER,

ipRouteMask

IpAddress,

ipRouteMetric5

INTEGER,

ipRouteInfo

OBJECT IDENTIFIER

}

ipRouteDest OBJECT-TYPE

SYNTAX  IpAddress

ACCESS  read-write

STATUS  mandatory

DESCRIPTION

"The destination IP address of this route.  An

entry with a value of 0.0.0.0 is considered a

default route.  Multiple routes to a single

destination can appear in the table, but access to

such multiple entries is dependent on the table-

access mechanisms defined by the network

management protocol in use."

::= { ipRouteEntry 1 }

ipRouteIfIndex OBJECT-TYPE

SYNTAX  INTEGER

ACCESS  read-write

STATUS  mandatory

DESCRIPTION

"The index value which uniquely identifies the

local interface through which the next hop of this

route should be reached.  The interface identified

by a particular value of this index is the same

interface as identified by the same value of

ifIndex."

::= { ipRouteEntry 2 }

ipRouteMetric1 OBJECT-TYPE

SYNTAX  INTEGER

ACCESS  read-write

STATUS  mandatory

DESCRIPTION

"The primary routing metric for this route.  The

semantics of this metric are determined by the

routing-protocol specified in the route's

ipRouteProto value.  If this metric is not used,

its value should be set to -1."

::= { ipRouteEntry 3 }

ipRouteMetric2 OBJECT-TYPE

SYNTAX  INTEGER

ACCESS  read-write

STATUS  mandatory

DESCRIPTION

"An alternate routing metric for this route.  The

semantics of this metric are determined by the

routing-protocol specified in the route's

ipRouteProto value.  If this metric is not used,

its value should be set to -1."

::= { ipRouteEntry 4 }

ipRouteMetric3 OBJECT-TYPE

SYNTAX  INTEGER

ACCESS  read-write

STATUS  mandatory

DESCRIPTION

"An alternate routing metric for this route.  The

semantics of this metric are determined by the

routing-protocol specified in the route's

ipRouteProto value.  If this metric is not used,

its value should be set to -1."

::= { ipRouteEntry 5 }

ipRouteMetric4 OBJECT-TYPE

SYNTAX  INTEGER

ACCESS  read-write

STATUS  mandatory

DESCRIPTION

"An alternate routing metric for this route.  The

semantics of this metric are determined by the

routing-protocol specified in the route's

ipRouteProto value.  If this metric is not used,

its value should be set to -1."

::= { ipRouteEntry 6 }

ipRouteNextHop OBJECT-TYPE

SYNTAX  IpAddress

ACCESS  read-write

STATUS  mandatory

DESCRIPTION

"The IP address of the next hop of this route.

(In the case of a route bound to an interface

which is realized via a broadcast media, the value

of this field is the agent's IP address on that

interface.)"

::= { ipRouteEntry 7 }

ipRouteType OBJECT-TYPE

SYNTAX  INTEGER {

other(1),        -- none of the following

invalid(2),      -- an invalidated route

-- route to directly

direct(3),       -- connected (sub-)network

-- route to a non-local

indirect(4)      -- host/network/sub-network

}

ACCESS  read-write

STATUS  mandatory

DESCRIPTION

"The type of route.  Note that the values

direct(3) and indirect(4) refer to the notion of

direct and indirect routing in the IP

architecture.

Setting this object to the value invalid(2) has

the effect of invalidating the corresponding entry

in the ipRouteTable object.  That is, it

effectively dissasociates the destination

identified with said entry from the route

identified with said entry.  It is an

implementation-specific matter as to whether the

agent removes an invalidated entry from the table.

Accordingly, management stations must be prepared

to receive tabular information from agents that

corresponds to entries not currently in use.

Proper interpretation of such entries requires

examination of the relevant ipRouteType object."

::= { ipRouteEntry 8 }

ipRouteProto OBJECT-TYPE

SYNTAX  INTEGER {

other(1),       -- none of the following

-- non-protocol information,

-- e.g., manually configured

local(2),       -- entries

-- set via a network

netmgmt(3),     -- management protocol

-- obtained via ICMP,

icmp(4),        -- e.g., Redirect

-- the remaining values are

-- all gateway routing

-- protocols

egp(5),

ggp(6),

hello(7),

rip(8),

is-is(9),

es-is(10),

ciscoIgrp(11),

bbnSpfIgp(12),

ospf(13),

bgp(14)

}

ACCESS  read-only

STATUS  mandatory

DESCRIPTION

"The routing mechanism via which this route was

learned.  Inclusion of values for gateway routing

protocols is not intended to imply that hosts

should support those protocols."

::= { ipRouteEntry 9 }

ipRouteAge OBJECT-TYPE

SYNTAX  INTEGER

ACCESS  read-write

STATUS  mandatory

DESCRIPTION

"The number of seconds since this route was last

updated or otherwise determined to be correct.

Note that no semantics of `too old' can be implied

except through knowledge of the routing protocol

by which the route was learned."

::= { ipRouteEntry 10 }

ipRouteMask OBJECT-TYPE

SYNTAX  IpAddress

ACCESS  read-write

STATUS  mandatory

DESCRIPTION

"Indicate the mask to be logical-ANDed with the

destination address before being compared to the

value in the ipRouteDest field.  For those systems

that do not support arbitrary subnet masks, an

agent constructs the value of the ipRouteMask by

determining whether the value of the correspondent

ipRouteDest field belong to a class-A, B, or C

network, and then using one of:

mask           network

255.0.0.0      class-A

255.255.0.0    class-B

255.255.255.0  class-C

If the value of the ipRouteDest is 0.0.0.0 (a

default route), then the mask value is also

0.0.0.0.  It should be noted that all IP routing

subsystems implicitly use this mechanism."

::= { ipRouteEntry 11 }

ipRouteMetric5 OBJECT-TYPE

SYNTAX  INTEGER

ACCESS  read-write

STATUS  mandatory

DESCRIPTION

"An alternate routing metric for this route.  The

semantics of this metric are determined by the

routing-protocol specified in the route's

ipRouteProto value.  If this metric is not used,

its value should be set to -1."

::= { ipRouteEntry 12 }

ipRouteInfo OBJECT-TYPE

SYNTAX  OBJECT IDENTIFIER

ACCESS  read-only

STATUS  mandatory

DESCRIPTION

"A reference to MIB definitions specific to the

particular routing protocol which is responsible

for this route, as determined by the value

specified in the route's ipRouteProto value.  If

this information is not present, its value should

be set to the OBJECT IDENTIFIER { 0 0 }, which is

a syntatically valid object identifier, and any

conformant implementation of ASN.1 and BER must be

able to generate and recognize this value."

::= { ipRouteEntry 13 }

-- the IP Address Translation table

-- The IP address translation table contain the IpAddress to

-- `physical' address equivalences.  Some interfaces do not

-- use translation tables for determining address

-- equivalences (e.g., DDN-X.25 has an algorithmic method);

-- if all interfaces are of this type, then the Address

-- Translation table is empty, i.e., has zero entries.

ipNetToMediaTable OBJECT-TYPE

SYNTAX  SEQUENCE OF IpNetToMediaEntry

ACCESS  not-accessible

STATUS  mandatory

DESCRIPTION

"The IP Address Translation table used for mapping

from IP addresses to physical addresses."

::= { ip 22 }

ipNetToMediaEntry OBJECT-TYPE

SYNTAX  IpNetToMediaEntry

ACCESS  not-accessible

STATUS  mandatory

DESCRIPTION

"Each entry contains one IpAddress to `physical'

address equivalence."

INDEX   { ipNetToMediaIfIndex,

ipNetToMediaNetAddress }

::= { ipNetToMediaTable 1 }

IpNetToMediaEntry ::=

SEQUENCE {

ipNetToMediaIfIndex

INTEGER,

ipNetToMediaPhysAddress

PhysAddress,

ipNetToMediaNetAddress

IpAddress,

ipNetToMediaType

INTEGER

}

ipNetToMediaIfIndex OBJECT-TYPE

SYNTAX  INTEGER

ACCESS  read-write

STATUS  mandatory

DESCRIPTION

"The interface on which this entry's equivalence

is effective.  The interface identified by a

particular value of this index is the same

interface as identified by the same value of

ifIndex."

::= { ipNetToMediaEntry 1 }

ipNetToMediaPhysAddress OBJECT-TYPE

SYNTAX  PhysAddress

ACCESS  read-write

STATUS  mandatory

DESCRIPTION

"The media-dependent `physical' address."

::= { ipNetToMediaEntry 2 }

ipNetToMediaNetAddress OBJECT-TYPE

SYNTAX  IpAddress

ACCESS  read-write

STATUS  mandatory

DESCRIPTION

"The IpAddress corresponding to the media-

dependent `physical' address."

::= { ipNetToMediaEntry 3 }

ipNetToMediaType OBJECT-TYPE

SYNTAX  INTEGER {

other(1),        -- none of the following

invalid(2),      -- an invalidated mapping

dynamic(3),

static(4)

}

ACCESS  read-write

STATUS  mandatory

DESCRIPTION

"The type of mapping.

Setting this object to the value invalid(2) has

the effect of invalidating the corresponding entry

in the ipNetToMediaTable.  That is, it effectively

dissasociates the interface identified with said

entry from the mapping identified with said entry.

It is an implementation-specific matter as to

whether the agent removes an invalidated entry

from the table.  Accordingly, management stations

must be prepared to receive tabular information

from agents that corresponds to entries not

currently in use.  Proper interpretation of such

entries requires examination of the relevant

ipNetToMediaType object."

::= { ipNetToMediaEntry 4 }

-- additional IP objects

ipRoutingDiscards OBJECT-TYPE

SYNTAX  Counter

ACCESS  read-only

STATUS  mandatory

DESCRIPTION

"The number of routing entries which were chosen

to be discarded even though they are valid.  One

possible reason for discarding such an entry could

be to free-up buffer space for other routing

entries."

::= { ip 23 }

-- the ICMP group

-- Implementation of the ICMP group is mandatory for all

-- systems.

icmpInMsgs OBJECT-TYPE

SYNTAX  Counter

ACCESS  read-only

STATUS  mandatory

DESCRIPTION

"The total number of ICMP messages which the

entity received.  Note that this counter includes

all those counted by icmpInErrors."

::= { icmp 1 }

icmpInErrors OBJECT-TYPE

SYNTAX  Counter

ACCESS  read-only

STATUS  mandatory

DESCRIPTION

"The number of ICMP messages which the entity

received but determined as having ICMP-specific

errors (bad ICMP checksums, bad length, etc.)."

::= { icmp 2 }

icmpInDestUnreachs OBJECT-TYPE

SYNTAX  Counter

ACCESS  read-only

STATUS  mandatory

DESCRIPTION

"The number of ICMP Destination Unreachable

messages received."

::= { icmp 3 }

icmpInTimeExcds OBJECT-TYPE

SYNTAX  Counter

ACCESS  read-only

STATUS  mandatory

DESCRIPTION

"The number of ICMP Time Exceeded messages

received."

::= { icmp 4 }

icmpInParmProbs OBJECT-TYPE

SYNTAX  Counter

ACCESS  read-only

STATUS  mandatory

DESCRIPTION

"The number of ICMP Parameter Problem messages

received."

::= { icmp 5 }

icmpInSrcQuenchs OBJECT-TYPE

SYNTAX  Counter

ACCESS  read-only

STATUS  mandatory

DESCRIPTION

"The number of ICMP Source Quench messages

received."

::= { icmp 6 }

icmpInRedirects OBJECT-TYPE

SYNTAX  Counter

ACCESS  read-only

STATUS  mandatory

DESCRIPTION

"The number of ICMP Redirect messages received."

::= { icmp 7 }

icmpInEchos OBJECT-TYPE

SYNTAX  Counter

ACCESS  read-only

STATUS  mandatory

DESCRIPTION

"The number of ICMP Echo (request) messages

received."

::= { icmp 8 }

icmpInEchoReps OBJECT-TYPE

SYNTAX  Counter

ACCESS  read-only

STATUS  mandatory

DESCRIPTION

"The number of ICMP Echo Reply messages received."

::= { icmp 9 }

icmpInTimestamps OBJECT-TYPE

SYNTAX  Counter

ACCESS  read-only

STATUS  mandatory

DESCRIPTION

"The number of ICMP Timestamp (request) messages

received."

::= { icmp 10 }

icmpInTimestampReps OBJECT-TYPE

SYNTAX  Counter

ACCESS  read-only

STATUS  mandatory

DESCRIPTION

"The number of ICMP Timestamp Reply messages

received."

::= { icmp 11 }

icmpInAddrMasks OBJECT-TYPE

SYNTAX  Counter

ACCESS  read-only

STATUS  mandatory

DESCRIPTION

"The number of ICMP Address Mask Request messages

received."

::= { icmp 12 }

icmpInAddrMaskReps OBJECT-TYPE

SYNTAX  Counter

ACCESS  read-only

STATUS  mandatory

DESCRIPTION

"The number of ICMP Address Mask Reply messages

received."

::= { icmp 13 }

icmpOutMsgs OBJECT-TYPE

SYNTAX  Counter

ACCESS  read-only

STATUS  mandatory

DESCRIPTION

"The total number of ICMP messages which this

entity attempted to send.  Note that this counter

includes all those counted by icmpOutErrors."

::= { icmp 14 }

icmpOutErrors OBJECT-TYPE

SYNTAX  Counter

ACCESS  read-only

STATUS  mandatory

DESCRIPTION

"The number of ICMP messages which this entity did

not send due to problems discovered within ICMP

such as a lack of buffers.  This value should not

include errors discovered outside the ICMP layer

such as the inability of IP to route the resultant

datagram.  In some implementations there may be no

types of error which contribute to this counter's

value."

::= { icmp 15 }

icmpOutDestUnreachs OBJECT-TYPE

SYNTAX  Counter

ACCESS  read-only

STATUS  mandatory

DESCRIPTION

"The number of ICMP Destination Unreachable

messages sent."

::= { icmp 16 }

icmpOutTimeExcds OBJECT-TYPE

SYNTAX  Counter

ACCESS  read-only

STATUS  mandatory

DESCRIPTION

"The number of ICMP Time Exceeded messages sent."

::= { icmp 17 }

icmpOutParmProbs OBJECT-TYPE

SYNTAX  Counter

ACCESS  read-only

STATUS  mandatory

DESCRIPTION

"The number of ICMP Parameter Problem messages

sent."

::= { icmp 18 }

icmpOutSrcQuenchs OBJECT-TYPE

SYNTAX  Counter

ACCESS  read-only

STATUS  mandatory

DESCRIPTION

"The number of ICMP Source Quench messages sent."

::= { icmp 19 }

icmpOutRedirects OBJECT-TYPE

SYNTAX  Counter

ACCESS  read-only

STATUS  mandatory

DESCRIPTION

"The number of ICMP Redirect messages sent.  For a

host, this object will always be zero, since hosts

do not send redirects."

::= { icmp 20 }

icmpOutEchos OBJECT-TYPE

SYNTAX  Counter

ACCESS  read-only

STATUS  mandatory

DESCRIPTION

"The number of ICMP Echo (request) messages sent."

::= { icmp 21 }

icmpOutEchoReps OBJECT-TYPE

SYNTAX  Counter

ACCESS  read-only

STATUS  mandatory

DESCRIPTION

"The number of ICMP Echo Reply messages sent."

::= { icmp 22 }

icmpOutTimestamps OBJECT-TYPE

SYNTAX  Counter

ACCESS  read-only

STATUS  mandatory

DESCRIPTION

"The number of ICMP Timestamp (request) messages

sent."

::= { icmp 23 }

icmpOutTimestampReps OBJECT-TYPE

SYNTAX  Counter

ACCESS  read-only

STATUS  mandatory

DESCRIPTION

"The number of ICMP Timestamp Reply messages

sent."

::= { icmp 24 }

icmpOutAddrMasks OBJECT-TYPE

SYNTAX  Counter

ACCESS  read-only

STATUS  mandatory

DESCRIPTION

"The number of ICMP Address Mask Request messages

sent."

::= { icmp 25 }

icmpOutAddrMaskReps OBJECT-TYPE

SYNTAX  Counter

ACCESS  read-only

STATUS  mandatory

DESCRIPTION

"The number of ICMP Address Mask Reply messages

sent."

::= { icmp 26 }

-- the TCP group

-- Implementation of the TCP group is mandatory for all

-- systems that implement the TCP.

-- Note that instances of object types that represent

-- information about a particular TCP connection are

-- transient; they persist only as long as the connection

-- in question.

tcpRtoAlgorithm OBJECT-TYPE

SYNTAX  INTEGER {

other(1),    -- none of the following

constant(2), -- a constant rto

rsre(3),     -- MIL-STD-1778, Appendix B

vanj(4)      -- Van Jacobson's algorithm [10]

}

ACCESS  read-only

STATUS  mandatory

DESCRIPTION

"The algorithm used to determine the timeout value

used for retransmitting unacknowledged octets."

::= { tcp 1 }

tcpRtoMin OBJECT-TYPE

SYNTAX  INTEGER

ACCESS  read-only

STATUS  mandatory

DESCRIPTION

"The minimum value permitted by a TCP

implementation for the retransmission timeout,

measured in milliseconds.  More refined semantics

for objects of this type depend upon the algorithm

used to determine the retransmission timeout.  In

particular, when the timeout algorithm is rsre(3),

an object of this type has the semantics of the

LBOUND quantity described in RFC 793."

::= { tcp 2 }

tcpRtoMax OBJECT-TYPE

SYNTAX  INTEGER

ACCESS  read-only

STATUS  mandatory

DESCRIPTION

"The maximum value permitted by a TCP

implementation for the retransmission timeout,

measured in milliseconds.  More refined semantics

for objects of this type depend upon the algorithm

used to determine the retransmission timeout.  In

particular, when the timeout algorithm is rsre(3),

an object of this type has the semantics of the

UBOUND quantity described in RFC 793."

::= { tcp 3 }

tcpMaxConn OBJECT-TYPE

SYNTAX  INTEGER

ACCESS  read-only

STATUS  mandatory

DESCRIPTION

"The limit on the total number of TCP connections

the entity can support.  In entities where the

maximum number of connections is dynamic, this

object should contain the value -1."

::= { tcp 4 }

tcpActiveOpens OBJECT-TYPE

SYNTAX  Counter

ACCESS  read-only

STATUS  mandatory

DESCRIPTION

"The number of times TCP connections have made a

direct transition to the SYN-SENT state from the

CLOSED state."

::= { tcp 5 }

tcpPassiveOpens OBJECT-TYPE

SYNTAX  Counter

ACCESS  read-only

STATUS  mandatory

DESCRIPTION

"The number of times TCP connections have made a

direct transition to the SYN-RCVD state from the

LISTEN state."

::= { tcp 6 }

tcpAttemptFails OBJECT-TYPE

SYNTAX  Counter

ACCESS  read-only

STATUS  mandatory

DESCRIPTION

"The number of times TCP connections have made a

direct transition to the CLOSED state from either

the SYN-SENT state or the SYN-RCVD state, plus the

number of times TCP connections have made a direct

transition to the LISTEN state from the SYN-RCVD

state."

::= { tcp 7 }

tcpEstabResets OBJECT-TYPE

SYNTAX  Counter

ACCESS  read-only

STATUS  mandatory

DESCRIPTION

"The number of times TCP connections have made a

direct transition to the CLOSED state from either

the ESTABLISHED state or the CLOSE-WAIT state."

::= { tcp 8 }

tcpCurrEstab OBJECT-TYPE

SYNTAX  Gauge

ACCESS  read-only

STATUS  mandatory

DESCRIPTION

"The number of TCP connections for which the

current state is either ESTABLISHED or CLOSE-

WAIT."

::= { tcp 9 }

tcpInSegs OBJECT-TYPE

SYNTAX  Counter

ACCESS  read-only

STATUS  mandatory

DESCRIPTION

"The total number of segments received, including

those received in error.  This count includes

segments received on currently established

connections."

::= { tcp 10 }

tcpOutSegs OBJECT-TYPE

SYNTAX  Counter

ACCESS  read-only

STATUS  mandatory

DESCRIPTION

"The total number of segments sent, including

those on current connections but excluding those

containing only retransmitted octets."

::= { tcp 11 }

tcpRetransSegs OBJECT-TYPE

SYNTAX  Counter

ACCESS  read-only

STATUS  mandatory

DESCRIPTION

"The total number of segments retransmitted - that

is, the number of TCP segments transmitted

containing one or more previously transmitted

octets."

::= { tcp 12 }

-- the TCP Connection table

-- The TCP connection table contains information about this

-- entity's existing TCP connections.

tcpConnTable OBJECT-TYPE

SYNTAX  SEQUENCE OF TcpConnEntry

ACCESS  not-accessible

STATUS  mandatory

DESCRIPTION

"A table containing TCP connection-specific

information."

::= { tcp 13 }

tcpConnEntry OBJECT-TYPE

SYNTAX  TcpConnEntry

ACCESS  not-accessible

STATUS  mandatory

DESCRIPTION

"Information about a particular current TCP

connection.  An object of this type is transient,

in that it ceases to exist when (or soon after)

the connection makes the transition to the CLOSED

state."

INDEX   { tcpConnLocalAddress,

tcpConnLocalPort,

tcpConnRemAddress,

tcpConnRemPort }

::= { tcpConnTable 1 }

TcpConnEntry ::=

SEQUENCE {

tcpConnState

INTEGER,

tcpConnLocalAddress

IpAddress,

tcpConnLocalPort

INTEGER (0..65535),

tcpConnRemAddress

IpAddress,

tcpConnRemPort

INTEGER (0..65535)

}

tcpConnState OBJECT-TYPE

SYNTAX  INTEGER {

closed(1),

listen(2),

synSent(3),

synReceived(4),

established(5),

finWait1(6),

finWait2(7),

closeWait(8),

lastAck(9),

closing(10),

timeWait(11),

deleteTCB(12)

}

ACCESS  read-write

STATUS  mandatory

DESCRIPTION

"The state of this TCP connection.

The only value which may be set by a management

station is deleteTCB(12).  Accordingly, it is

appropriate for an agent to return a `badValue'

response if a management station attempts to set

this object to any other value.

If a management station sets this object to the

value deleteTCB(12), then this has the effect of

deleting the TCB (as defined in RFC 793) of the

corresponding connection on the managed node,

resulting in immediate termination of the

connection.

As an implementation-specific option, a RST

segment may be sent from the managed node to the

other TCP endpoint (note however that RST segments

are not sent reliably)."

::= { tcpConnEntry 1 }

tcpConnLocalAddress OBJECT-TYPE

SYNTAX  IpAddress

ACCESS  read-only

STATUS  mandatory

DESCRIPTION

"The local IP address for this TCP connection.  In

the case of a connection in the listen state which

is willing to accept connections for any IP

interface associated with the node, the value

0.0.0.0 is used."

::= { tcpConnEntry 2 }

tcpConnLocalPort OBJECT-TYPE

SYNTAX  INTEGER (0..65535)

ACCESS  read-only

STATUS  mandatory

DESCRIPTION

"The local port number for this TCP connection."

::= { tcpConnEntry 3 }

tcpConnRemAddress OBJECT-TYPE

SYNTAX  IpAddress

ACCESS  read-only

STATUS  mandatory

DESCRIPTION

"The remote IP address for this TCP connection."

::= { tcpConnEntry 4 }

tcpConnRemPort OBJECT-TYPE

SYNTAX  INTEGER (0..65535)

ACCESS  read-only

STATUS  mandatory

DESCRIPTION

"The remote port number for this TCP connection."

::= { tcpConnEntry 5 }

-- additional TCP objects

tcpInErrs OBJECT-TYPE

SYNTAX  Counter

ACCESS  read-only

STATUS  mandatory

DESCRIPTION

"The total number of segments received in error

(e.g., bad TCP checksums)."

::= { tcp 14 }

tcpOutRsts OBJECT-TYPE

SYNTAX  Counter

ACCESS  read-only

STATUS  mandatory

DESCRIPTION

"The number of TCP segments sent containing the

RST flag."

::= { tcp 15 }

-- the UDP group

-- Implementation of the UDP group is mandatory for all

-- systems which implement the UDP.

udpInDatagrams OBJECT-TYPE

SYNTAX  Counter

ACCESS  read-only

STATUS  mandatory

DESCRIPTION

"The total number of UDP datagrams delivered to

UDP users."

::= { udp 1 }

udpNoPorts OBJECT-TYPE

SYNTAX  Counter

ACCESS  read-only

STATUS  mandatory

DESCRIPTION

"The total number of received UDP datagrams for

which there was no application at the destination

port."

::= { udp 2 }

udpInErrors OBJECT-TYPE

SYNTAX  Counter

ACCESS  read-only

STATUS  mandatory

DESCRIPTION

"The number of received UDP datagrams that could

not be delivered for reasons other than the lack

of an application at the destination port."

::= { udp 3 }

udpOutDatagrams OBJECT-TYPE

SYNTAX  Counter

ACCESS  read-only

STATUS  mandatory

DESCRIPTION

"The total number of UDP datagrams sent from this

entity."

::= { udp 4 }

-- the UDP Listener table

-- The UDP listener table contains information about this

-- entity's UDP end-points on which a local application is

-- currently accepting datagrams.

udpTable OBJECT-TYPE

SYNTAX  SEQUENCE OF UdpEntry

ACCESS  not-accessible

STATUS  mandatory

DESCRIPTION

"A table containing UDP listener information."

::= { udp 5 }

udpEntry OBJECT-TYPE

SYNTAX  UdpEntry

ACCESS  not-accessible

STATUS  mandatory

DESCRIPTION

"Information about a particular current UDP

listener."

INDEX   { udpLocalAddress, udpLocalPort }

::= { udpTable 1 }

UdpEntry ::=

SEQUENCE {

udpLocalAddress

IpAddress,

udpLocalPort

INTEGER (0..65535)

}

udpLocalAddress OBJECT-TYPE

SYNTAX  IpAddress

ACCESS  read-only

STATUS  mandatory

DESCRIPTION

"The local IP address for this UDP listener.  In

the case of a UDP listener which is willing to

accept datagrams for any IP interface associated

with the node, the value 0.0.0.0 is used."

::= { udpEntry 1 }

udpLocalPort OBJECT-TYPE

SYNTAX  INTEGER (0..65535)

ACCESS  read-only

STATUS  mandatory

DESCRIPTION

"The local port number for this UDP listener."

::= { udpEntry 2 }

-- the EGP group

-- Implementation of the EGP group is mandatory for all

-- systems which implement the EGP.

egpInMsgs OBJECT-TYPE

SYNTAX  Counter

ACCESS  read-only

STATUS  mandatory

DESCRIPTION

"The number of EGP messages received without

error."

::= { egp 1 }

egpInErrors OBJECT-TYPE

SYNTAX  Counter

ACCESS  read-only

STATUS  mandatory

DESCRIPTION

"The number of EGP messages received that proved

to be in error."

::= { egp 2 }

egpOutMsgs OBJECT-TYPE

SYNTAX  Counter

ACCESS  read-only

STATUS  mandatory

DESCRIPTION

"The total number of locally generated EGP

messages."

::= { egp 3 }

egpOutErrors OBJECT-TYPE

SYNTAX  Counter

ACCESS  read-only

STATUS  mandatory

DESCRIPTION

"The number of locally generated EGP messages not

sent due to resource limitations within an EGP

entity."

::= { egp 4 }

-- the EGP Neighbor table

-- The EGP neighbor table contains information about this

-- entity's EGP neighbors.

egpNeighTable OBJECT-TYPE

SYNTAX  SEQUENCE OF EgpNeighEntry

ACCESS  not-accessible

STATUS  mandatory

DESCRIPTION

"The EGP neighbor table."

::= { egp 5 }

egpNeighEntry OBJECT-TYPE

SYNTAX  EgpNeighEntry

ACCESS  not-accessible

STATUS  mandatory

DESCRIPTION

"Information about this entity's relationship with

a particular EGP neighbor."

INDEX   { egpNeighAddr }

::= { egpNeighTable 1 }

EgpNeighEntry ::=

SEQUENCE {

egpNeighState

INTEGER,

egpNeighAddr

IpAddress,

egpNeighAs

INTEGER,

egpNeighInMsgs

Counter,

egpNeighInErrs

Counter,

egpNeighOutMsgs

Counter,

egpNeighOutErrs

Counter,

egpNeighInErrMsgs

Counter,

egpNeighOutErrMsgs

Counter,

egpNeighStateUps

Counter,

egpNeighStateDowns

Counter,

egpNeighIntervalHello

INTEGER,

egpNeighIntervalPoll

INTEGER,

egpNeighMode

INTEGER,

egpNeighEventTrigger

INTEGER

}

egpNeighState OBJECT-TYPE

SYNTAX  INTEGER {

idle(1),

acquisition(2),

down(3),

up(4),

cease(5)

}

ACCESS  read-only

STATUS  mandatory

DESCRIPTION

"The EGP state of the local system with respect to

this entry's EGP neighbor.  Each EGP state is

represented by a value that is one greater than

the numerical value associated with said state in

RFC 904."

::= { egpNeighEntry 1 }

egpNeighAddr OBJECT-TYPE

SYNTAX  IpAddress

ACCESS  read-only

STATUS  mandatory

DESCRIPTION

"The IP address of this entry's EGP neighbor."

::= { egpNeighEntry 2 }

egpNeighAs OBJECT-TYPE

SYNTAX  INTEGER

ACCESS  read-only

STATUS  mandatory

DESCRIPTION

"The autonomous system of this EGP peer.  Zero

should be specified if the autonomous system

number of the neighbor is not yet known."

::= { egpNeighEntry 3 }

egpNeighInMsgs OBJECT-TYPE

SYNTAX  Counter

ACCESS  read-only

STATUS  mandatory

DESCRIPTION

"The number of EGP messages received without error

from this EGP peer."

::= { egpNeighEntry 4 }

egpNeighInErrs OBJECT-TYPE

SYNTAX  Counter

ACCESS  read-only

STATUS  mandatory

DESCRIPTION

"The number of EGP messages received from this EGP

peer that proved to be in error (e.g., bad EGP

checksum)."

::= { egpNeighEntry 5 }

egpNeighOutMsgs OBJECT-TYPE

SYNTAX  Counter

ACCESS  read-only

STATUS  mandatory

DESCRIPTION

"The number of locally generated EGP messages to

this EGP peer."

::= { egpNeighEntry 6 }

egpNeighOutErrs OBJECT-TYPE

SYNTAX  Counter

ACCESS  read-only

STATUS  mandatory

DESCRIPTION

"The number of locally generated EGP messages not

sent to this EGP peer due to resource limitations

within an EGP entity."

::= { egpNeighEntry 7 }

egpNeighInErrMsgs OBJECT-TYPE

SYNTAX  Counter

ACCESS  read-only

STATUS  mandatory

DESCRIPTION

"The number of EGP-defined error messages received

from this EGP peer."

::= { egpNeighEntry 8 }

egpNeighOutErrMsgs OBJECT-TYPE

SYNTAX  Counter

ACCESS  read-only

STATUS  mandatory

DESCRIPTION

"The number of EGP-defined error messages sent to

this EGP peer."

::= { egpNeighEntry 9 }

egpNeighStateUps OBJECT-TYPE

SYNTAX  Counter

ACCESS  read-only

STATUS  mandatory

DESCRIPTION

"The number of EGP state transitions to the UP

state with this EGP peer."

::= { egpNeighEntry 10 }

egpNeighStateDowns OBJECT-TYPE

SYNTAX  Counter

ACCESS  read-only

STATUS  mandatory

DESCRIPTION

"The number of EGP state transitions from the UP

state to any other state with this EGP peer."

::= { egpNeighEntry 11 }

egpNeighIntervalHello OBJECT-TYPE

SYNTAX  INTEGER

ACCESS  read-only

STATUS  mandatory

DESCRIPTION

"The interval between EGP Hello command

retransmissions (in hundredths of a second).  This

represents the t1 timer as defined in RFC 904."

::= { egpNeighEntry 12 }

egpNeighIntervalPoll OBJECT-TYPE

SYNTAX  INTEGER

ACCESS  read-only

STATUS  mandatory

DESCRIPTION

"The interval between EGP poll command

retransmissions (in hundredths of a second).  This

represents the t3 timer as defined in RFC 904."

::= { egpNeighEntry 13 }

egpNeighMode OBJECT-TYPE

SYNTAX  INTEGER { active(1), passive(2) }

ACCESS  read-only

STATUS  mandatory

DESCRIPTION

"The polling mode of this EGP entity, either

passive or active."

::= { egpNeighEntry 14 }

egpNeighEventTrigger OBJECT-TYPE

SYNTAX  INTEGER { start(1), stop(2) }

ACCESS  read-write

STATUS  mandatory

DESCRIPTION

"A control variable used to trigger operator-

initiated Start and Stop events.  When read, this

variable always returns the most recent value that

egpNeighEventTrigger was set to.  If it has not

been set since the last initialization of the

network management subsystem on the node, it

returns a value of `stop'.

When set, this variable causes a Start or Stop

event on the specified neighbor, as specified on

pages 8-10 of RFC 904.  Briefly, a Start event

causes an Idle peer to begin neighbor acquisition

and a non-Idle peer to reinitiate neighbor

acquisition.  A stop event causes a non-Idle peer

to return to the Idle state until a Start event

occurs, either via egpNeighEventTrigger or

otherwise."

::= { egpNeighEntry 15 }

-- additional EGP objects

egpAs OBJECT-TYPE

SYNTAX  INTEGER

ACCESS  read-only

STATUS  mandatory

DESCRIPTION

"The autonomous system number of this EGP entity."

::= { egp 6 }

-- the Transmission group

-- Based on the transmission media underlying each interface

-- on a system, the corresponding portion of the Transmission

-- group is mandatory for that system.

-- When Internet-standard definitions for managing

-- transmission media are defined, the transmission group is

-- used to provide a prefix for the names of those objects.

-- Typically, such definitions reside in the experimental

-- portion of the MIB until they are "proven", then as a

-- part of the Internet standardization process, the

-- definitions are accordingly elevated and a new object

-- identifier, under the transmission group is defined. By

-- convention, the name assigned is:

--

--     type OBJECT IDENTIFIER    ::= { transmission number }

--

-- where "type" is the symbolic value used for the media in

-- the ifType column of the ifTable object, and "number" is

-- the actual integer value corresponding to the symbol.

-- the SNMP group

-- Implementation of the SNMP group is mandatory for all

-- systems which support an SNMP protocol entity.  Some of

-- the objects defined below will be zero-valued in those

-- SNMP implementations that are optimized to support only

-- those functions specific to either a management agent or

-- a management station.  In particular, it should be

-- observed that the objects below refer to an SNMP entity,

-- and there may be several SNMP entities residing on a

-- managed node (e.g., if the node is hosting acting as

-- a management station).

snmpInPkts OBJECT-TYPE

SYNTAX  Counter

ACCESS  read-only

STATUS  mandatory

DESCRIPTION

"The total number of Messages delivered to the

SNMP entity from the transport service."

::= { snmp 1 }

snmpOutPkts OBJECT-TYPE

SYNTAX  Counter

ACCESS  read-only

STATUS  mandatory

DESCRIPTION

"The total number of SNMP Messages which were

passed from the SNMP protocol entity to the

transport service."

::= { snmp 2 }

snmpInBadVersions OBJECT-TYPE

SYNTAX  Counter

ACCESS  read-only

STATUS  mandatory

DESCRIPTION

"The total number of SNMP Messages which were

delivered to the SNMP protocol entity and were for

an unsupported SNMP version."

::= { snmp 3 }

snmpInBadCommunityNames OBJECT-TYPE

SYNTAX  Counter

ACCESS  read-only

STATUS  mandatory

DESCRIPTION

"The total number of SNMP Messages delivered to

the SNMP protocol entity which used a SNMP

community name not known to said entity."

::= { snmp 4 }

snmpInBadCommunityUses OBJECT-TYPE

SYNTAX  Counter

ACCESS  read-only

STATUS  mandatory

DESCRIPTION

"The total number of SNMP Messages delivered to

the SNMP protocol entity which represented an SNMP

operation which was not allowed by the SNMP

community named in the Message."

::= { snmp 5 }

snmpInASNParseErrs OBJECT-TYPE

SYNTAX  Counter

ACCESS  read-only

STATUS  mandatory

DESCRIPTION

"The total number of ASN.1 or BER errors

encountered by the SNMP protocol entity when

decoding received SNMP Messages."

::= { snmp 6 }

-- { snmp 7 } is not used

snmpInTooBigs OBJECT-TYPE

SYNTAX  Counter

ACCESS  read-only

STATUS  mandatory

DESCRIPTION

"The total number of SNMP PDUs which were

delivered to the SNMP protocol entity and for

which the value of the error-status field is

`tooBig'."

::= { snmp 8 }

snmpInNoSuchNames OBJECT-TYPE

SYNTAX  Counter

ACCESS  read-only

STATUS  mandatory

DESCRIPTION

"The total number of SNMP PDUs which were

delivered to the SNMP protocol entity and for

which the value of the error-status field is

`noSuchName'."

::= { snmp 9 }

snmpInBadValues OBJECT-TYPE

SYNTAX  Counter

ACCESS  read-only

STATUS  mandatory

DESCRIPTION

"The total number of SNMP PDUs which were

delivered to the SNMP protocol entity and for

which the value of the error-status field is

`badValue'."

::= { snmp 10 }

snmpInReadOnlys OBJECT-TYPE

SYNTAX  Counter

ACCESS  read-only

STATUS  mandatory

DESCRIPTION

"The total number valid SNMP PDUs which were

delivered to the SNMP protocol entity and for

which the value of the error-status field is

`readOnly'.  It should be noted that it is a

protocol error to generate an SNMP PDU which

contains the value `readOnly' in the error-status

field, as such this object is provided as a means

of detecting incorrect implementations of the

SNMP."

::= { snmp 11 }

snmpInGenErrs OBJECT-TYPE

SYNTAX  Counter

ACCESS  read-only

STATUS  mandatory

DESCRIPTION

"The total number of SNMP PDUs which were

delivered to the SNMP protocol entity and for

which the value of the error-status field is

`genErr'."

::= { snmp 12 }

snmpInTotalReqVars OBJECT-TYPE

SYNTAX  Counter

ACCESS  read-only

STATUS  mandatory

DESCRIPTION

"The total number of MIB objects which have been

retrieved successfully by the SNMP protocol entity

as the result of receiving valid SNMP Get-Request

and Get-Next PDUs."

::= { snmp 13 }

snmpInTotalSetVars OBJECT-TYPE

SYNTAX  Counter

ACCESS  read-only

STATUS  mandatory

DESCRIPTION

"The total number of MIB objects which have been

altered successfully by the SNMP protocol entity

as the result of receiving valid SNMP Set-Request

PDUs."

::= { snmp 14 }

snmpInGetRequests OBJECT-TYPE

SYNTAX  Counter

ACCESS  read-only

STATUS  mandatory

DESCRIPTION

"The total number of SNMP Get-Request PDUs which

have been accepted and processed by the SNMP

protocol entity."

::= { snmp 15 }

snmpInGetNexts OBJECT-TYPE

SYNTAX  Counter

ACCESS  read-only

STATUS  mandatory

DESCRIPTION

"The total number of SNMP Get-Next PDUs which have

been accepted and processed by the SNMP protocol

entity."

::= { snmp 16 }

snmpInSetRequests OBJECT-TYPE

SYNTAX  Counter

ACCESS  read-only

STATUS  mandatory

DESCRIPTION

"The total number of SNMP Set-Request PDUs which

have been accepted and processed by the SNMP

protocol entity."

::= { snmp 17 }

snmpInGetResponses OBJECT-TYPE

SYNTAX  Counter

ACCESS  read-only

STATUS  mandatory

DESCRIPTION

"The total number of SNMP Get-Response PDUs which

have been accepted and processed by the SNMP

protocol entity."

::= { snmp 18 }

snmpInTraps OBJECT-TYPE

SYNTAX  Counter

ACCESS  read-only

STATUS  mandatory

DESCRIPTION

"The total number of SNMP Trap PDUs which have

been accepted and processed by the SNMP protocol

entity."

::= { snmp 19 }

snmpOutTooBigs OBJECT-TYPE

SYNTAX  Counter

ACCESS  read-only

STATUS  mandatory

DESCRIPTION

"The total number of SNMP PDUs which were

generated by the SNMP protocol entity and for

which the value of the error-status field is

`tooBig.'"

::= { snmp 20 }

snmpOutNoSuchNames OBJECT-TYPE

SYNTAX  Counter

ACCESS  read-only

STATUS  mandatory

DESCRIPTION

"The total number of SNMP PDUs which were

generated by the SNMP protocol entity and for

which the value of the error-status is

`noSuchName'."

::= { snmp 21 }

snmpOutBadValues OBJECT-TYPE

SYNTAX  Counter

ACCESS  read-only

STATUS  mandatory

DESCRIPTION

"The total number of SNMP PDUs which were

generated by the SNMP protocol entity and for

which the value of the error-status field is

`badValue'."

::= { snmp 22 }

-- { snmp 23 } is not used

snmpOutGenErrs OBJECT-TYPE

SYNTAX  Counter

ACCESS  read-only

STATUS  mandatory

DESCRIPTION

"The total number of SNMP PDUs which were

generated by the SNMP protocol entity and for

which the value of the error-status field is

`genErr'."

::= { snmp 24 }

snmpOutGetRequests OBJECT-TYPE

SYNTAX  Counter

ACCESS  read-only

STATUS  mandatory

DESCRIPTION

"The total number of SNMP Get-Request PDUs which

have been generated by the SNMP protocol entity."

::= { snmp 25 }

snmpOutGetNexts OBJECT-TYPE

SYNTAX  Counter

ACCESS  read-only

STATUS  mandatory

DESCRIPTION

"The total number of SNMP Get-Next PDUs which have

been generated by the SNMP protocol entity."

::= { snmp 26 }

snmpOutSetRequests OBJECT-TYPE

SYNTAX  Counter

ACCESS  read-only

STATUS  mandatory

DESCRIPTION

"The total number of SNMP Set-Request PDUs which

have been generated by the SNMP protocol entity."

::= { snmp 27 }

snmpOutGetResponses OBJECT-TYPE

SYNTAX  Counter

ACCESS  read-only

STATUS  mandatory

DESCRIPTION

"The total number of SNMP Get-Response PDUs which

have been generated by the SNMP protocol entity."

::= { snmp 28 }

snmpOutTraps OBJECT-TYPE

SYNTAX  Counter

ACCESS  read-only

STATUS  mandatory

DESCRIPTION

"The total number of SNMP Trap PDUs which have

been generated by the SNMP protocol entity."

::= { snmp 29 }

snmpEnableAuthenTraps OBJECT-TYPE

SYNTAX  INTEGER { enabled(1), disabled(2) }

ACCESS  read-write

STATUS  mandatory

DESCRIPTION

"Indicates whether the SNMP agent process is

permitted to generate authentication-failure

traps.  The value of this object overrides any

configuration information; as such, it provides a

means whereby all authentication-failure traps may

be disabled.

Note that it is strongly recommended that this

object be stored in non-volatile memory so that it

remains constant between re-initializations of the

network management system."

::= { snmp 30 }

coldStart TRAP-TYPE

ENTERPRISE  snmp

DESCRIPTION

"A coldStart trap signifies that the sending

protocol entity is reinitializing itself such

that the agent's configuration or the rotocol

entity implementation may be altered."

::= 0

warmStart TRAP-TYPE

ENTERPRISE  snmp

DESCRIPTION

"A warmStart trap signifies that the sending

protocol entity is reinitializing itself such

that neither the agent configuration nor the

protocol entity implementation is altered."

::= 1

linkDown TRAP-TYPE

ENTERPRISE  snmp

VARIABLES   { ifIndex }

DESCRIPTION

"A linkDown trap signifies that the sending

protocol entity recognizes a failure in one of

the communication links represented in the

agent's configuration."

::= 2

linkUp TRAP-TYPE

ENTERPRISE  snmp

VARIABLES   { ifIndex }

DESCRIPTION

"A linkUp trap signifies that the sending

protocol entity recognizes that one of the

communication links represented in the agent's

configuration has come up."

::= 3

authenticationFailure TRAP-TYPE

ENTERPRISE  snmp

DESCRIPTION

"An authenticationFailure trap signifies that

the sending protocol entity is the addressee

of a protocol message that is not properly

authenticated.  While implementations of the

SNMP must be capable of generating this trap,

they must also be capable of suppressing the

emission of such traps via an implementation-

specific mechanism."

::= 4

END