-- groups in MIB-II

system       OBJECT IDENTIFIER ::= { mib-2 1 }

interfaces   OBJECT IDENTIFIER ::= { mib-2 2 }

at           OBJECT IDENTIFIER ::= { mib-2 3 }

ip           OBJECT IDENTIFIER ::= { mib-2 4 }

icmp         OBJECT IDENTIFIER ::= { mib-2 5 }

tcp          OBJECT IDENTIFIER ::= { mib-2 6 }

udp          OBJECT IDENTIFIER ::= { mib-2 7 }

egp          OBJECT IDENTIFIER ::= { mib-2 8 }

-- historical (some say hysterical)

-- cmot      OBJECT IDENTIFIER ::= { mib-2 9 }

transmission OBJECT IDENTIFIER ::= { mib-2 10 }

snmp         OBJECT IDENTIFIER ::= { mib-2 11 }

-- the System group

-- Implementation of the System group is mandatory for all

-- systems.  If an agent is not configured to have a value

-- for any of these variables, a string of length 0 is returned.

sysDescr OBJECT-TYPE

SYNTAX  DisplayString (SIZE (0..255))

ACCESS  read-only

STATUS  mandatory

DESCRIPTION

"A textual description of the entity.  This value

should include the full name and version

identification of the system's hardware type,

software operating-system, and networking

software.  It is mandatory that this only contain

printable ASCII characters."

::= { system 1 }

sysObjectID OBJECT-TYPE

SYNTAX  OBJECT IDENTIFIER

ACCESS  read-only

STATUS  mandatory

DESCRIPTION

"The vendor's authoritative identification of the

network management subsystem contained in the

entity.  This value is allocated within the SMI

enterprises subtree (1.3.6.1.4.1) and provides an

easy and unambiguous means for determining `what

kind of box' is being managed.  For example, if

vendor `Flintstones, Inc.' was assigned the

subtree 1.3.6.1.4.1.4242, it could assign the

identifier 1.3.6.1.4.1.4242.1.1 to its `Fred Router'."

::= { system 2 }

sysUpTime OBJECT-TYPE

SYNTAX  TimeTicks

ACCESS  read-only

STATUS  mandatory

DESCRIPTION

"The time (in hundredths of a second) since the

network management portion of the system was last

re-initialized."

::= { system 3 }

sysContact OBJECT-TYPE

SYNTAX  DisplayString (SIZE (0..255))

ACCESS  read-write

STATUS  mandatory

DESCRIPTION

"The textual identification of the contact person

for this managed node, together with information

on how to contact this person."

::= { system 4 }

sysName OBJECT-TYPE

SYNTAX  DisplayString (SIZE (0..255))

ACCESS  read-write

STATUS  mandatory

DESCRIPTION

"An administratively-assigned name for this

managed node.  By convention, this is the node's

fully-qualified domain name."

::= { system 5 }

sysLocation OBJECT-TYPE

SYNTAX  DisplayString (SIZE (0..255))

ACCESS  read-write

STATUS  mandatory

DESCRIPTION

"The physical location of this node (e.g.,

`telephone closet, 3rd floor')."

::= { system 6 }

sysServices OBJECT-TYPE

SYNTAX  INTEGER (0..127)

ACCESS  read-only

STATUS  mandatory

DESCRIPTION

"A value which indicates the set of services that

this entity primarily offers.

The value is a sum.  This sum initially takes the

value zero, Then, for each layer, L, in the range

1 through 7, that this node performs transactions

for, 2 raised to (L - 1) is added to the sum.  For

example, a node which performs primarily routing

functions would have a value of 4 (2^(3-1)).  In

contrast, a node which is a host offering

application services would have a value of 72

(2^(4-1) + 2^(7-1)).  Note that in the context of

the Internet suite of protocols, values should be

calculated accordingly:

layer  functionality

1  physical (e.g., repeaters)

2  datalink/subnetwork (e.g., bridges)

3  internet (e.g., IP gateways)

4  end-to-end  (e.g., IP hosts)

7  applications (e.g., mail relays)

For systems including OSI protocols, layers 5 and

6 may also be counted."

::= { system 7 }